COST Technical Committee

“Telecommunications, Information Science and Technology”
(TC-TIST)

# COST Action 289

Spectrum and Power Efficient Broadband Communications

# PROGRESS REPORT

Period: from 1 July 2005 to 30 June 2006

Start date of the Action: 24 April 2003	    Last update: 15 June 2006

- OVERVIEW: ACTION IDENTIFICATION DATA

##### COST Action 289

##### Title: Spectrum and Power Efficient Broadband Communications

TC Recommendation: 8 October 2002	First MC meeting: 24 March 2003

CSO Approval: 2 December 2002		Last MC meeting: 23-24 March 2006

Start date: 23 April 2003			Final Report:

Duration: 48 months				Evaluation Report:

Extension:					TC Evaluation:

End date: 22 April 2007

Number of signatories: 19

Signatories and date of signature:

| Belgium | 05/08/2003 | Confirmed |
| --- | --- | --- |
| Bulgaria | 04/04/2003 | Confirmed |
| Czech Republic | 05/08/2003 | Confirmed |
| France | 05/08/2003 | Confirmed |
| Germany | 02/04/2003 | Confirmed |
| Greece | 30/09/2003 | Confirmed |
| Hungary | 28/03/2003 | Confirmed |
| Italy | 05/08/2003 | Confirmed |
| Malta | 11/03/2005 | Confirmed |
| Norway | 05/08/2003 | Confirmed |
| Portugal | 21/09/2004 | Confirmed |
| Romania | 23/06/2004 | Confirmed |
| Serbia and Montenegro | 20/11/2003 | Confirmed |
| Slovakia | 14/04/2003 | Confirmed |
| Spain | 25/09/2003 | Confirmed |
| Sweden | 12/08/2003 | Confirmed |
| Switzerland | 23/05/2003 | Confirmed |
| Turkey | 23/04/2003 | Confirmed |
| UK | 18/10/2004 | Confirmed |

Institutes of non-COST countries: None

Area: Mobile Communications

Action web site: http://cost289.ee.hacettepe.edu.tr

Chairperson: Prof. Dr. Mehmet Safak

| Prof. Mehmet Safak Hacettepe University Dept. of Electrical and Electronics Eng. 06532 Beytepe, Ankara, Turkey | Tel: +90-312-297 70 70 Fax: +90-312-299 21 25 E-mail: msafak@hacettepe.edu.tr http://www.ee.hacettepe.edu.tr |
| --- | --- |

TC Rapporteur: 	Prof. G. Brussaard

External Evaluator:

1.   OBJECTIVES

The requirements for higher user mobility and for ever increasing data rates seem to act as two major drivers of future communication systems. Based on this observation, the main objective of this Action is to increase the capacity of communication systems with constraints on the transmission bandwidth and the transmitter power, bearing in mind the cost effectiveness and the practicality of the solution. This imposes serious requirements on communication systems calling for higher data rates, higher mobility and, at the same time, a less hazardous electromagnetic environment. To achieve this goal, existing and innovative communication techniques and systems need to be investigated from the viewpoint of the bandwidth and the power efficiency.

This project will contribute to the realization of spectrum- and power-efficient integrated multimedia communications, thus to minimize the bandwidth requirements for high data rate services. Reciprocally, the requirements on the transmit power will be reduced for a given transmission bandwidth. The resulting benefits will include the provision of higher data rates, larger coverage areas, higher mobility to the users, and longer battery life for mobile terminals. All these will contribute to the provision of better QoS and cost effective services. The systems thus designed will also contribute to the creation of a less polluted electromagnetic propagation environment. Consequently, one may expect reduced electromagnetic compatibility (EMC) problems with other electronic systems and significant reduction in hazardous radiation effects to biological systems. The outcome of the Action is believed to provide valuable contribution to the literature and the technology in the related areas.

- TECHNICAL DESCRIPTION AND IMPLEMENTATION

The philosophy of the Action is based on close co-operation between the scientists and engineers from the telecommunication industry and the academia. In agreement with the above-cited objectives, the research efforts will be focused on the problems discussed below.

  - Working Groups

The research activities of this Action are conducted in two Working Groups (WG’s):

- WG1: Information Theoretical Description of Radio Systems

The aim of WG1 is to provide an analytical description of mobile systems and related architectures, and to carry out detailed analyses of modern communication systems. The emphasis will be on mathematical studies and simulations rather than on the implementation issues. The output of WG1 may also be used as an input to WG2.

A list of research areas of the WG1 includes, but not limited to:

- Spectral efficiency and power efficiency

- Channel capacity

- User capacity

- Modulation

- Coding

- WG2: Communications Techniques and Systems

The objective of WG2 is to study the implementation of techniques and algorithms for increasing the data rates, the user mobility, the reliability as well as the power- and spectrum-efficiency of communication systems.

The research activities of the WG2 include, but not limited to:

- Adaptive transmission techniques

- Software defined radio

- Adaptive/reconfigurable networks

- Multicarrier systems

- Multiple access techniques

- Multiuser detection

- Multiple-input multiple-output (MIMO) systems

The two WGs define the framework of the research activities proposed in this Action. Management Committee (MC) decided to monitor the scientific activities in the two WGs in plenary sessions. Consequently, the MC has decided not to appoint heads to these WGs.

  - Project Groups

The MC believes that a COST Action should provide a platform not only for the presentation of individual research results of the participants but should also act as a means to create synergy in coordinated and cooperative research efforts in order to find coherent and practical solutions to the problem formulated in the MoU.

Accordingly, the MC decided to form the following three project groups, with active participation of a number of institutions listed below. The detailed description of these projects is given in Section 4: RESULTS.

a)  Pervasive wireless access for 4G

Coordinator: Prof. Armin Wittneben, ETH Zurich

The participating organizations: ETH Zurich, Hacettepe University, Norwegian University of Science and Technology, University of Ulm, Budapest University of Technology and Economics, University Carlos III of Madrid.

- Wide are coverage and high mobility access systems for 4G

Coordinator:  Prof. Arne Svensson, Chalmers University of Technology

The participating organizations: Chalmers University of Technology, University of Florence, Ramon Llull University, CEI-LETI, University Carlos III of Madrid, Hacettepe University, TU Kosice, Czech Academy of Science, Norwegian University of Science and Technology, DLR.

- Software defined radio

Coordinator: Prof. Sandor Imre, Budapest University of Technology and Economics.

The participating organizations: Budapest University of Technology and Economics, University Carlos III of Madrid, Ramon Llull University, Universitat Politècnica de Catalunya, University of Athens.

The above research projects were finalized during the 4th MCM, held in Zurich, 15-16 March 2004. Each researcher is free to choose his/her own individual research area, within the framework of the project. The researchers participating to the above projects decided to cooperate closely and concentrate their efforts to investigate in detail and bring coherent solutions to the formulated problems.

It was desired to organise an inauguration meeting for the above three project groups with the participation of young researchers. However, in view of the large numbers of participation in each of the projects, the project coordinators prepared a description of their project programmes and disseminated these programmes to the members.

Some of the researchers visited the project coordinators in STSMs for better coordination of the research efforts. These researchers are also encouraged to attend the Workshops organized in order to increase their awareness about the project, to present their research results and for further coordination and cooperation.

Three separate e-mail groups are formed for the three research projects, namely Pervasive Wireless Access for 4G, Wide Area Coverage Systems with High Mobility for 4G, and SDR. These e-mail groups are aimed for exchange of information and documents between the young researchers directly involved in the projects and to provide a common platform for discussing the outstanding items.

- PARTICIPATION AND COORDINATION

  - Management Committee

Chairman:

Prof. Dr. Mehmet Safak

Hacettepe University

Department of Electrical and Electronics Engineering

06800 Beytepe, Ankara, Turkey

Tel: +90-312-297 70 70

Fax: +90-312-299 21 25

E-mail: msafak@hacettepe.edu.tr

Vice-Chairman:

Prof. Dr. Hermann Rohling

Technical University of Hamburg-Harburg

Arbeitsbereich Nachrichtentechnik
Eissendorfer Str. 40 D-21073 Hamburg

Tel (+4940) 42878-3028
Fax (+4940) 42878-2281
E-mail rohling@tu-harburg.de

Secretary:

Serap Hasimoğlu-Ertas

Hacettepe University
Department of Electrical and Electronics Engineering
06800 Beytepe, Ankara, Turkey

Tel: +90-312-297 70 00 
Fax: +90-312-299 21 25
E-mail: serap@ee.hacettepe.edu.tr

Members:

Prof. H. Rohling, Technical University of Hamburg-Harburg, Germany

Prof. A. Wittneben, Swiss Federal Institute of Technology, Switzerland:

Prof. J. Lindner, University of Ulm, Germany

Prof. H. Vinck, University of Essen, Germany

Prof. A. Svensson, Chalmers University of Technology, Sweden

Dr. S. Kaiser, German Aerospace Centre, DLR, Germany

Prof. L. Vandendorpe, Universite Catholique de Louvain, Belgium

Researcher M. Dangl, University of Ulm, Germany

Prof. A. Polydoros, University of Athens, Greece

Researcher K. Nikitopoulos, University of Athens, Greece

Prof. D. Kocur, Technical University of Kosice, Slovakia

Prof. A. Ahlen, Uppsala University, Sweden

Prof. L. Pap, Budapest University of Technology and Economics, Hungary:
Prof. S. Imre, Budapest University of Technology and Economics, Hungary

Researcher G. Jeney, Budapest Un. of Technology and Economics, Hungary

Prof. R. Fantacci, University of Florence, Italy

Researcher Ing. D. Tarchi, University of Florence, Italy

###### Prof. J. L. Pijoan, Ramon Llull University, Spain

Dr. J. P. Romero, Polytechnic University of Catalunya, Spain

Researcher F.A. Freixer, Polytechnic University of Catalunya, Spain

Dr. A. Garcia-Armada, University Carlos III of Madrid, Spain

Researcher M. P. S. Fernandez, University Carlos III of Madrid, Spain

Researcher V. P. G. Jimenez, University Carlos III of Madrid, Spain

Prof. M. Safak, Hacettepe University, Turkey

Researcher G. Pay, Hacettepe University, Turkey

Dr. G. Meyer, Swiss Federal Institute of Technology, Switzerland

Prof. G. E. Oien, Department of Telematics, NTNU, Norway

Prof. K. J. Hole, Department of Telematics, NTNU, Norway

Dr. J. Simsa, Academy of Sciences, Czech Republic

Prof. G. Iliev, Technical University of Sofia, Bulgaria

Prof. Dr. V. Bota, Technical University of Cluj-Napoca, Romania

Prof. Dr. A. de Sabato, Politchnical University of Timisoara

Dr. H. Sari, SUPELEC, France

Dr. R. Visoz, France Telecom R& D, France

Prof. D. Bajic, University of Novi Sad, Serbia and Montenegro

Dr. W. Teich, University of Ulm, Germany

Dr. U.C. Fiebig, DLR, Germany

Dr. C. Toker, Hacettepe University, Turkey

Dr. S. Mayrargue, CEA-LETI, France

Dr. K. Hamdi, UMIST, UK

Dr. S. Morosi, University of Florence, Italy

Dr. A. Berthet, SUPELEC, France

Dr. M. Drutarovsky, Technical University of Kosice, Slovakia

Dr. E. Alsusa, University of Manchaster, UK

Dr. J. Azzopardi, Acrosslimits, Malta

Prof. S. Robert, EIVD, Switzerland

Prof. A. Safak, Baskent University, Turkey

Dr. E. Aktas, Hacettepe University, Turkey

Researcher S. Plass, German Aerospace Centre, DLR, Germany

Researcher S. Sand, German Aerospace Centre, DLR, Germany

Researcher D. Vukobratovic, University of Novi Sad, Serbia and Montenegro

Researcher F. Chiti, University of Florence, Italy

Researcher G. Mennuti, University of Florence, Italy

Researcher S. Wendt, France Telecom R&D, France

Researcher P. Katsis, Aristotle University of Thessaloniki, Greece

Researcher G. Papadopoulos, Aristotle University of Thessaloniki, Greece

Researcher S. Hasimoglu-Ertas, Hacettepe University, Turkey

Researcher F. Balasz, Budapest Un. of Technology and Economics, Hungary

Researcher G. Rabai, Budapest Un. of Technology and Economics, Hungary

Researcher C. Vilella, Ramon Llull University, Spain

Researcher M. Deumal, Ramon Llull University, Spain

Researcher S. Berger, ETH Zurich, Switzerland

Researcher I. Hammerstroem, ETH Zurich, Switzerland

Researcher B. Rankov, ETH Zurich, Switzerland

Researcher J. Cizova, Technical University of Kosice, Slovakia

Researcher P. Galajda, Technical University of Kosice, Slovakia

Researcher V.Senk, University of Novi Sad, Serbia and Montenegro

Researcher S.Kethulle, Norwegian Univ. of Science and Technology, Norway

Researcher I. Safak, Hacettepe University, Turkey

Researcher M. Cakir, Hacettepe University, Turkey

Researcher J. Krajnak, TU Kosice, Slovakia

Researcher X. Reves, Polytechnic University of Catalunya, Spain

Researcher P. Bergada, Ramon Llull University, Spain

Researcher Z.Andreas, NKUA University of Athens, Greece

Researcher M. Varga, Technical University of Cluj-Napoca, Romania

Researcher L.Wischhof, TU Hamburg, Germany

Researcher P. Fazekas Budapest Univ. of Technology and Economics, Hungary

Researcher V. Hassel, Norwegian Univ. of Science and Technology, Norway

Researcher A. Papaioannou, Aristotle University of Thessaloniki, Greece

Researcher P. Zsolt, TU Cluj-Napoca, Romania

Researcher  R. Aquilue, Ramon Llull University, Spain

Researcher R. M.Alsina, Ramon Llull University, Spain

Researcher  I. Gutierrez, Ramon Llull University, Spain

Researcher V.Markovic, Polytechnic University of Catalunya, Spain

Researcher F. Adelantado, Polytechnic University of Catalunya, Spain

Researcher L. Alonso, Polytechnic University of Catalunya, Spain

Researcher I. Cosovic, DLR, Germany

Researcher A. Dammann, DLR, Germany

Researcher S. Brandes, DLR, Germany

Researcher S. Gligorevic, DLR, Germany

Researcher E. Haas, DLR, Germany

Researcher D. Yacoub, University of Ulm, Germany

Researcher I. Perisa, University of Ulm, Germany

Researcher W. Zhang, University of Ulm, Germany

Researcher A. Silva, Institute of Telecommunications, Portugal

Researcher C. Stefanovic, University of Novi-Sad, Serbia and Montenegro

Researcher M. Dikmen, Hacettepe University, Turkey

Researcher Ö. Karabacak, Hacettepe University, Turkey

Researcher K. Baumgartner, University of Applied Sciences (EIVD)

Researcher Cedomir Stefanovic, Univ. of Novi Sad, Serbia and Montenegro

Researcher Marc Deumal Herraiz,  Ramon Llull University, Spain

  - Participating Institutions

Belgium	 			Universite Catholique de Louvain

Bulgaria				Technical University of Sofia

Czech Republic			Academy of Sciencies

France				Supelec

France Telecom R&D

CEA-CETI

Germany	 			Technical University of Hamburg-Harburg

University of Ulm

University of Essen

German Aerospace Centre, DLR

Greece				University of Athens

Aristotle University of Thessaloniki

Hungary	 			Budapest Univ. of Technology and Economics
Italy				University of Florence

###### Malta				AcrossLimits

Norway				Department of Telematics, NTNU

###### Portugal				Uni. of Aveiro, Inst. of Telecommunications

Romania				Technical University of Cluj-Napoca

Politechnic University of Timisoara

Serbia and Montenegro		University of Novi Sad

Slovak Republic			Technical University of Kosice

###### Spain				Ramon Llull University

Polytechnic University of Catalunya

University Carlos III of Madrid

Sweden				Chalmers University of Technology

Uppsala University

Switzerland			Swiss Federal Institute of Technology, Zurich

University of Applied Sciences (EIVD)

Turkey				Hacettepe University

United Kingdom			University of Manchester

  - Meetings of the Management Committee

Inaug. MCM	24 March 2003, Brussels, Belgium

2. MCM		3-4 July 2003, Hamburg, Germany

3. MCM 		30-31 October 2003, Kosice, Slovak Republic

4. MCM		15-16 March 2004, Zurich, Switzerland

5. MCM		7-9 July 2004, Budapest, Hungary (jointly with 1st Workshop)

6. MCM 		28-29 October 2004, Barcelona, Spain

7. MCM		7-8 March 2005, Oberpfaffenhofen, Munich Germany

8. MCM		6-8 July 2005, Antalya, Turkey (jointly with 2nd Workshop)

9. MCM		3-4 November 2005, Madrid, Spain

10. MCM		23-24 March 2006, Novi Sad, Serbia and Montenegro

  - Meetings of the Working Groups

N/A.

  - Short-term scientific missions

The STSMs already realized/scheduled are as follows:

  - Wide Area Coverage for 4G:

Host: Prof. Arne Svensson, Chalmers University of Technology, Göteborg

Visitors:

Marc Deumal-Herraiz (Ramon Llull University, 13.9.2004 - 29.11, 2004),

Victor Pedro Gil-Jimenez (Univ. Carlos III of Madrid, 4.9.2004-9.12, 2004),

Simon Plass (DLR, 20.9.2004 - 24.9. 2004),

Jozef Krajnak (TU Kosice, 18.9.2004 - 3.10. 2004),

Serap Hasimoglu-Ertas (Hacettepe University, 20.9.2004 - 17.12, 2004)

Host: Prof. Hermann Rohling, TUHH, Hamburg, Germany

Visitor: M.Varga (Technical University Cluj-Napoca, Romania, 13.06.2005-24.06.2005)

Host: Prof. J.L. Pijoan, Ramon Llull University, Barcelona

Visitors:

Ali Behravan (Chalmers University of Technology, 20 Feb.-12 March 2006)

Pavol Pavelka (Technical University of Kosice, 5-9 June 2006)

  - Pervasive Access for 4G:

Host: Prof. Armin Wittneben, ETH Zurich

Visitor: Doris Yacoub (University of Ulm, 11 -17 June 2006)

  - Software Defined Radio:

Host: 	Polytechnic University of Catalunya (UPC), Barcelona

Visitors:

Ferenc Balazs (BUTE, 24.10.2004 – 4.11, 2004)

Pavol Galajda (TU Kosice, 18.10.2004 – 22.10, 2004)

The researchers, who are supported by the STSM programme, are strongly recommended to report to the MC about the scientific results of their STSMs.

In order to bring a uniformity to financial supports for the STSMs, the MC decided to follow the following payment method: The cost of plane ticket will be deducted from 2000 €, which is the maximum financial support for an STSM, and the remaining amount will be divided into 30 days, which represents the duration of maximum stay in a STSM. This will give the amount of financial support per day. The Chairman is authorized to increase or decrease the amount of daily support, thus found, up to 20%, depending on the living conditions of the host country. The total financial support will be given by the sum of the cost of the transportation ticket and the number of nights spent in STSM times the daily financial support.

- RESULTS

The project on Wide Area Coverage and high mobility access systems for 4G addresses the problems of centralized systems with high mobility, thus with correspondingly lower data rates but wider coverage areas. The project on Pervasive Wireless Access for 4G is mainly concerned with decentralized networks with lower mobility, much higher data rates, and consequently restricted coverage areas. The SDR project aims to bridge these two projects horizontally since the implementation of the systems coexisting in the same platform are essential for the reconfigurability and adaptivity purposes.

These projects are believed to be sufficiently flexible so that each researcher can choose his particular research topic and thus make the best use of his/her expertise. The individual research results are believed to provide valuable contributions for the solution of the particular project.

The research efforts resulted in 107 presentations and 13 tutorials in the 10 MCMs organised so far. The 107 presentations made during the MCMs and the COST289 Workshops, 50 journal papers, book chapters and books and 216 conference papers are available in the Action web site.

The above-cited publications originate from

- the individual research efforts of the participants not necessarily related to one the three projects cited-above;

- the individual research results based on one of the three projects;

- the joint research efforts between various research teams on the three projects.

The number of publications belonging to the latter category, though presently not as high as desired, is believed to be increasing. Nevertheless, there are considerable number of individual contributions on one of the defined projects.

The STSMs supported by the Action greatly helped the creation of an atmosphere of cooperation. Consequently, there are already several papers presented during the 2nd COST289 Workshop and it is believed that this process will lead to some considerable joint publications. Similarly, some interesting papers are expected for the 3rd COST 289 Workshop to be held in Aveiro, Portugal, 12-14 July 2006.

4.1. Wide Area Coverage and High Mobility Access Systems for 4G

Intense research efforts are currently ongoing towards the definition of physical layer for 4G systems, which should provide services comparable to those offered by wired networks for a variety of applications such as interactive multimedia, voice over IP, network games, and video conferencing [1]. Trends are such that the 4G systems will be based on IP technology and deployed in a heterogeneous environment of various access networks, both wired and wireless. There are strong requirements for high data rates, high mobility, and flexible technologies. The traffic load of these systems will likely be dominated by the bursty data traffic.

There are many aspects of 4G systems, including detection techniques, antenna systems, adaptive modulation and coding, multiple access (MA), hybrid ARQ techniques, admission control, power control etc., to be studied in detail [2]. Since very high data rates are required, transmission system will very likely be based on multicarrier technology [3]. There is a close relationship between the choices of the appropriate transmission and MA techniques. MA implies the efficient sharing of the available bandwidth among a large number of users. The MA techniques currently used in 2G/3G, including those based on TDMA and DS-CDMA and possible combinations of the two, are suitable for voice communications but not necessarily for the bursty data traffic [1].

Several proposals have already been made towards the definition of 4G systems; these are mostly based on OFDM and CDMA. A national project in Sweden has suggested an adaptive multiplexing and OFDM transmission system for the downlink [4][5]. NTT DoCoMo has suggested a scheme based on the combination of OFDM and CDMA [6].

In the downlink, the signals are multiplexed within each cell, while it is multiple access between cells only. In the uplink, the situation is much more complicated since a combination of multiplexing and MA takes place in each terminal when more than one service is transmitted at the same time. The uplink is normally asynchronous and oscillators in different terminals are not synchronized. It may thus not be easy to use OFDMA in the uplinks due to its sensitivity to frequency synchronization errors. It may also be more difficult to use channel state information at the transmitter in the uplink, at least in FDD systems, due to the increased overhead. Currently, the intense research efforts are ongoing as to the relative advantages of various transmission, multiple access and duplexing techniques  [7]-[13].

For the 4G system uplinks, the following DoCoMo proposal may be used as a reference:

- Available uplink bandwidth is 40 MHz

- Carrier frequency likely around 5 GHz

- Maximum speed is 250 km/h

Note that the data rate requirement for high mobility by ITU is 100 Mbps, which might be difficult to achieve in a 40 MHz bandwidth.

The DoCoMo proposal for the downlink parameters is given by:

- Available downlink bandwidth is 100 MHz

- Carrier frequency likely around 5 GHz

- Maximum speed is 250 km/h

Further comparisons need to be carried out between OFDM- and CDMA-based systems for the downlink. A fair comparison between these systems in a multi-cellular environment with all the complexity taken into account is believed to be still missing.

All performance evaluations should be done using the widely accepted UMTS channel models. Preferably, performance should be evaluated in a multi-cell environment, but single cell performance and single link performance is acceptable as a starting point.

In this project, members of the Action focused their efforts to design and analyze physical layer solutions for 4G up- and down-links. Here, the interest is in systems with wide area coverage and high mobility, and there is no restriction on the choice  of transmission and MA techniques. So, in principle, any physical layer technique can be used and combined in such a way that high bandwidth- and power-efficient up- and down-links are obtained.

Five young researchers were hosted by Chalmers University of Technology on STSM during 2004 to establish a common understanding and cooperation on the research undertaken. These efforts resulted in the submission of several papers to the 2nd COST289 Workshop,  held in Antalya, during 6-7 July 2005. The following can be cited amongst the current topics related to this research project:

- Channel prediction and feedback information in OFDM-based adaptive modulation systems for 4G..

- Investigation of multiplexing and diversity techniques in 4G-downlinks by using adaptive MIMO systems.

- Effects of channel on multiuser CFO estimation for interleaved OFDMA uplink

- User-resource allocation methods for adaptive OFDM downlinks of mobile transmissions

The University of Carlos III of Madrid cooperates with Chalmers University of Technology in the research “Channel prediction and feedback information for adaptive modulation in wide area networks". This cooperation was established as a result of the STSM by Victor Pedro Gil-Jimenez from University Carlos III of Madrid to Chalmers University of Technology during 4.9.2004 - 9.12.2004. Similarly, Ali Behrevan from Chalmers University of Technology visited Ramon Llull University during 20 February-12 March 2006. As a result of this cooperation, the following conference paper was published: M.Deumal, J.L. Pijoan, I.Gutiérrez, A.Behravan, "Peak reduction of multi-carrier systems by Controlled Spectral Outgrowth", Proc. IEEE International conference on Acoustics, Speech and Signal Processing (ICASSP 2006), May 2006, Toulouse. The publication of some other papers is anticipated.

Research teams are currently working on the adaptive transmission technologies for 4G uplink and downlink design. Some of titles of publications in related areas are:

- Evaluations of a 4G Uplink System Based on Adaptive Single-Carrier TDMA

- Performances of Convolutionally-Coded QAM Mapping Non-Coded Bits

- Turbo-Equalization for Single-Carrier Transmission over MIMO Broadband Wireless Channel: Performance/Compexity Trade-Offs

- Frequency-Domain Techniques in Wireless Communications: OFDM, Precoded OFDM and SCT/FDE

### References

- A. Jamalipour, T. Wada and T. Yamazato, A tutorial on multiple access technologies for beyond 3G mobile networks, IEEE Communications Magazine, February 2005,  pp.110-117.

- R. Fantacci, F. Chiti, G. Mennuti, S. Morosi and D. Tarchi, Perspectives for present and future CDMA-based communication systems, IEEE Communications Magazine, February 2005, pp.95-100.

- Z. Wang and G.B. Giannakis, Wireless Multicarrier Communications, where Fourier meets Shannon, IEEE Signal Processing Magazine, pp. 29-48, May 2003.

- M. Sternad, T. Ottosson, A. Ahlén, A. Svensson, “Attaining both coverage and high spectral efficiency with adaptive OFDMA downlinks,” in Proc. IEEE VTC 2003 Fall, Orlando, USA, October 2003.

- W. Wang, T. Ottosson, M. Sternad, A. Ahlén, A. Svensson, “Impact of multiuser diversity and channel variability on adaptive OFDM,” in Proc. IEEE VTC 2003 Fall, Orlando, USA, October 2003.

- H. Atarashi, N. Maeda, S. Abeta, M. Sawahashi, “Broadband packet wireless access based on VSF-OFCDM and MC/DS-CDMA,” in Proc. IEEE PIMRC 2002, Lisbon, Portugal, pp. 992-997, Sept. 2002.

- P.Xia, S. Zhou and G.B. Giannakis, Bandwidth- and power-efficient multicarrier multiple access, IEEE Trans. Communications, vol.51, no.11, pp.1828-1837, Nov. 2003.

- H. Rohling and D. Galda, OFDM Transmission Technique: a strong candidate for next-generation mobile communications, Radio Science Bulletin, no.310, pp.47-58, Sept.2004.

- L.-L. Yang and L.Hanzo, Multicarrier DS-CDMA: A multiple access scheme foe ubiquitous broadband wireless communications, IEEE Communications Magazine, Oct. 2003, pp. 116-124.

- L.-L. Yang and L. Hanzo, Performance of generalized multicarrier DS-CDMA over Nakagami-m fading channels, IEEE Trans. Communications, vol. 50, no.6, June 2002, pp. 956-966

- J. Mintzer and P.A. Hoeher, Boosting the performance of wireless communication systems: Theory and practice of multiple-antenna techniques,  IEEE Communications Magazine, Oct.2004, pp.40-47.

- Q. H. Spencer, C.B. Peel, A.L.Swindlehurst, and M. Haardt, An introduction to the multi-user MIMO downlink, IEEE Communications Magazine, Oct. 2004, pp.60-67.

- A. Ghosh, D.R. Wolter, J.G. Andrews and R.Chen, Broadband wireless access with WiMax/802.16: Current performance benchmarks and future potential, IEEE Communications Magazine, Feb. 2005, pp.129-136.

4.2. Pervasive Wireless Access for 4G

Next generation WLANs will provide ubiquitous wireless connectivity for a variety of heterogeneous nodes, e.g., RFID tags for object identification, sensors and computers, with data rate requirements ranging from 1Mbps to 1Gbps. This breed of networks is referred to as pervasive wireless access networks, which are believed to have a tremendous impact on our lifestyle. It is anticipated that, in the future, pervasive wireless access systems will generate the bulk of the data traffic in the Internet.

For complexity reasons, low-end user nodes will have a single antenna. High-end user nodes will feature multiple antennas to improve throughput and coverage. The extended use and range of deployment will lead to a high node density. This makes cooperative signaling schemes an extremely attractive option. Channel adaptive scheduling, adaptive modulation and spatial multiplexing (MIMO) will be indispensable to achieve the required scalability and spectral efficiency. The heterogeneous QoS requirements would facilitate channel adaptive scheduling, but typical user nodes in wireless local area networks (WLANs) have low mobility and the channel may not vary sufficiently over time to make fair channel adaptive scheduling efficient.

For spectral reasons, next generation WLANs will operate beyond 5 GHz (e.g. at 17/24GHz). Here we face a poor scattering/rich array situation as opposed to the rich scattering/poor array situation at 5GHz [1]. Recent work [2]-[4] has shown the potential of cooperative signalling (in particular linear multi-node relaying) to exploit spatial multiplexing on poor scattering channels.

The main motivation of this project is to open up the benefits of cooperative diversity, channel adaptive scheduling and spatial multiplexing (MIMO) in a low mobility environment with poor scattering and heterogeneous nodes. This area is essentially unexplored to date and requires fundamental new work ranging from multinode/multihop channel modeling to efficient cooperative spatial multiplexing, cooperative medium access and cooperative channel adaptive scheduling schemes. In this project, attention will be given to linear but possibly time-variant processing at the cooperating nodes. This has the major advantage that the cooperation is transparent to an adaptive modulation scheme (symbol alphabet) used by the source. We are explicitly interested in schemes, that allow a seamless integration of multi-antenna nodes and that remain efficient in a poor scattering environment.

This project includes a number of deliverables, which will be produced by the participants of the project group (multihop/multinode channel model implementation, cooperative space-time code design criteria and evaluation software, cooperative WLAN simulation framework). The proposed research will benefit substantially from the RACOON Laboratory and the testbed infrastructure [5][6], which is currently available in ETH Zurich to COST289 researchers for verification and demonstration activities. RACOON constitutes a heavy investment of ETH Zurich in cooperative wireless communications.

The following can be cited amongst the papers presented in this topic during the 2nd COST289 Workshop, held in Antalya, 6-7 July 2005.

- Cooperative distributed multiuser MMSE relaying in wireless ad-hoc networks

- Temporal fairness enhanced scheduling for cooperative relaying networks in low mobility environments

- Cellular fixed relay networks: State of the art and open issues.

In addition to the above-cited papers, especially the research group in ETH Zurich is publishing extensively in this area in international conferences and journals (see references 177-189 in Annex I, Conference Papers)

References

[1] 	A. Wittneben, B. Rankov, MIMO Signaling for Low Rank Channels, URSI International Symposium on Electromagnetic Theory (2004 URSI EMT-S), Pisa, Italy (invited paper).

[2] 	A. Wittneben and B. Rankov, Distributed Antenna Arrays versus Cooperative Linear Relaying for Broadband Indoor MIMO Wireless, International Conference on Electromagnetics in Advanced Applications, ICEAA'03, Torino, Italy, Sept. 2003, (invited paper).

[3]	M. Kuhn, I. Hammerstroem, and A. Wittneben, Linear Scalable Dispersion Codes for Frequency Selective Channels, Proc. of the 8th International OFDM-Workshop, Sept. 2003.

[4] 	A. Wittneben and B. Rankov, Impact of Cooperative Relays on the Capacity of Rank-Deficient MIMO Channels, Proceedings of the 12th IST Summit on Mobile and Wireless Communications, Aveiro, Portugal, pp. 421-425, June 2003.

[5] 	R. Kueng, STORADIO, Repeater RF Unit, Specification, Design, Test, Version 1.1, 14.10.2001. Elektrobit, Bubikon.

[6] 	M. Wattinger, STORADIO, Storage Unit, Specification, Version 1.0, 17.10.2003, Elektrobit, Bubikon.

4.3. Software Defined Radio

The adaptive modulation and coding techniques require logical functions, coordination mechanisms and systems architectures better suited for adaptation. On the other hand, nowadays wireless systems access a great number of networks operating according to different standards. Inter-system roaming (e.g. GSM-IS95) and vertical handover (e.g. UMTS-WLAN) would require the implementation of many different standards in a single radio terminal/base station.

In order to avoid the usage of multi-mode terminals, the so-called software defined radio (SDR) concept has been proposed. Reconfigurable equipment with universal hardware and downloaded software can solve roaming/handover problem efficiently. Furthermore, modifications or even new techniques related to the air interface can be realized without effort enabling cost-efficient system upgrades. Therefore, SDR fits into the newly emerging “reconfigurable networks” concept.

The SDR implies a radio with enough programmability, thus easy and inexpensive modification of the signal processing steps, and thereby provides different communication schemes. Thanks to the developments in digital signal processing (DSP), field programmable gate arrays (FPGAs), application specific integrated circuits (ASICs) and digital signal processors (DSPs), we now have new options for the SDR. The ability to work across a wide range of frequency spectrum is believed to be a requirement for next generation SDR’s. Hardware architectures, for both baseband and RF sides, continue to improve. The SDR technology continues to evolve and is expected to be one of the key technologies for the future systems.

Within COST 289 the following research topics are considered:

- Hardware strategies to efficiently design reconfigurable radios (implementation  independent methods).

The efforts can be focused in:

  - The physical layer organization (distributed-DSP or centralized INTEL-type approaches) [1], [2]

  - More general approaches, including higher layer protocols, networking, services, interfaces, call admission control, etc

- Software strategies for downloading software modules and efficient reconfiguration algorithms.

- Specific hardware architectures that allow reconfiguration with a minimum power consumption penalty [3] or dynamic adaptation to the variations in user traffic.

- Experiences that partially implement multiple standards as GSM, EDGE, WCDMA-FDD, Bluetooth using some strategies on any platform ([2], [4], [5], etc.)

The targets can be summarized as follows::

- Starting from the Hardware Abstraction Layer presented by UPC, different hardware platforms may be integrated to that layer. The goal will be to have a useful, hardware-independent tool for programming real-time algorithms.

- Implementation of the critical parts of a communication system in FPGA, DSP and GPP.

A project in this area is involved with the implementation of a 3rd Generation Partnership Project (3GPP) M-PSK modulated turbo coded system in a multipath Rayleigh fading channel on a TMS320C6416 Digital Signal Processor (DSP) Starter Kit (DSK). The turbo decoder coprocessor (TCP) integrated into the Texas Instruments DSP is employed. The MATLAB link for Code Composer Studio (CCS) is used to acquire outputs from the DSP. The performance of such a system was measured in AWGN channel.

- Elaboration of a report on Software Defined Radio techniques for the implementation of the most critical aspects of multicarrier and MIMO receivers [6]. This topic is believed not covered in detail in the literature.

- A white paper may be prepared surveying the state of the art concepts and solutions for reconfigurable air interfaces [7].

- Studies in the field of RRM/CRRM about the possibility to manage the resources on terminals with flexible software structures (e.g. with a HAL) and the impact this may have on the QoS in heterogeneous networks.

In addition to the above research activities, the SDR project aims to cooperate with the other two projects, namely Pervasive Wireless Access for 4G and Wide Area Coverage and High Mobility Access Systems for 4G, which consider vertical approaches in wireless networks in two different environments (low versus high mobility, centralized versus distributed). The SDR project provides a bridge these two COST 289 projects horizontally, concentrating in the reconfigurability of different systems.

The researchers came together in a STSM from 25-29 October 2004 in UPC, Barcelona, to discuss the potential research topics and possibilities of cooperation.

Prof. J.L. Pijoan, from Ramon Llull University, Barcelona gave an invited seminar entitled Software Radio: An Enabling Technology for Mobile Communications, during 1st COST289 Workshop, held in Budapest, 7-9 July 2004. Prof. A. Polydoros gave an invited seminar entitled Radio Flexibility at the PHY Layer, during the 2nd COST289 Workshop, held in Antalya, 6-7 July 2005.

References

- S. Srikanteswara, J.H Reed, P. Athanas; Boyle, R, A soft radio architecture for reconfigurable platforms, IEEE Communications Magazine, vol. 38, no. 2, pp. 140–147, Feb. 2000.

- S. Srikanteswara, J. Neel, J.H Reed, P. Athanas, Soft radio implementations for 3G and future high data rate systems, IEEE Global Telecommunications Conference, GLOBECOM 2001, 25-29 November 2001, vol. 6, pp. 3370–3374.

- S. Srikanteswara, R.C. Palat, J.H Reed, P. Athanas, An overview of configurable computing machines for software radio handsets, IEEE Communications Magazine, vol. 41, no. 7, pp. 134–141, July 2003.

- A. A. Kountouris, C. Moy, L. Rambaud, P. Le Corre, A software radio approach for the transceiver transition from 2G to 2.5G to 3G, Sixth International, Symposium on Signal Processing and its Applications, 13-16 Aug. 2001, vol. 2, pp. 485–488.

- F. Jondral, A. Wiesler, R. Machauer, A software defined radio structure for 2nd and 3rd generation mobile communications standards, 2000 IEEE Sixth International Symposium on Spread Spectrum Techniques and Applications, 6-8 Sept. 2000, vol. 2, pp. 637-640.

[6] 	W.Xiang, T. Pratt and X. Wang, A software radio testbed for two-transmitter two-receiver space-time coding OFDM wireless LAN, IEEE Communications Magazine, vol.42, issue 6, June 2004, pp.S20-S28.

[7]	Z. Salcic and C. F. Mecklenbrauker, Software radio-Architectural Requirements, Research and Development Challenges, The 8th  Int. Conf. on Communication Systems, ICCS 2002, vol.2, 25-28 Nov. 2002, pp.711-716.

[8]	A. Perez-Neira, X.Mestre and J.R.Fonollosa, Smart antennas in software radio base stations, IEEE Communications Magazine, Feb. 2001, pp.166-173.

- DISSEMINATION OF RESULTS

  - Presentations in MCMs and Workshops

The number of participants, presentations and tutorial made during the MCMs held so far is listed below:

| MCM | Participants | Presentations | Tutorials |
| --- | --- | --- | --- |
| 1st MCM in Brussels, 24 March 2003 | 12 |  |  |
| 2nd MCM in Hamburg, 3-4 July 2003 | 16 | 7 |  |
| 3rd MCM, Kosice, 30-31 October 2003 | 25 | 8 |  |
| 4th MCM, Zurich, 15-16 March 2004 | 30 | 12 |  |
| 5th MCM and 1st COST 289 Workshop in Budapest, 7-9 July 2004 | 40 | 20 | 4 |
| 6th MCM in Barcelona, 28-29 October 2004 | 30 | 11 | 1 |
| 7th MCM in 7-8 March 2005, Oberpfaffenhofen/Munich | 35 | 6 | 1 |
| 8th MCM in 6-8 July 2005, Antalya | 46 | 21 | 4 |
| 9th MCM in 3-4 November 2005, Madrid | 32 | 12 | 1 |
| 10th MCM in 23-24 March 2006, Novi Sad | 32 | 12 | 2 |

  - Conferences and Workshops

    - 1st COST289 Workshop

The 1st COST289 Workshop was organized in Budapest during 7-9 July 2004 jointly with the 5th MCM.  The Workshop aimed to increase the spirit of cooperation between the young researchers from the COST nations; coordinate their research efforts under the guidance of the project coordinators; and to improve their background scientific level about the joint research projects.

The following seminars were given by the leading scientists in their area of research:

- Dr. Hiroyuki Atarashi, DoCoMo, Broadband Packet Wireless Access and its Field Experiments.

DoCoMo is one of the institutions heavily involved in 4G systems and has a proposal for the specifications for the uplink and downlinks. Dr. Atarashi presented uplink and downlink design proposals for 4G systems and discussed their relative advantages.The DoCoMo proposes the use of CDMA with variable spreading in time and frequency domains for the uplink.

- Prof. Lajos Hanzo, University of Southampton, Recital on Multicarrier Communications: Space-Time Coded Versus Adaptive OFDM/MC-CDMA.

MC-CDMA and OFDMA are strong contenders for the 4G systems. Prof. Hanzo discussed in detail the relative advantages of these systems.

- Prof. Armin Wittneben, ETH Zurich, Challenges in Pervasive Wireless Access

Prof. Wittneben gave an overview on the potential areas of applications, the state-of-art and trends in pervasive access systems.

- Prof. Joan Lluis Pijoan, Ramon Llull University, Software Radio: An Enabling Technology for Mobile Communications.

Prof. Pijoan presented a review of research activities about SDR and discussed the potential areas of cooperation between the research teams.

    - 2nd COST289 Workshop

The 2nd COST289 Workshop Special Topics on 4G Systems was organized in Antalya during 6-7 July 2005 jointly with the 8th MCM. Invited lectures by leading scientists provided the participants with the latest information and knowledge about technical and system aspects of spectrum and power-efficient-broadband communications. These lectures were followed by the presentations given by young researchers, thus creating an environment of discussion and cross-fertilization among the participants.

The following invited lectures were given during the 2nd COST 289 Workshop:

- Prof. Armin Wittneben, ETH Zurich; Recent Theoretical and Experimental Results in Multiuser Zero Forcing Relaying

Prof. Wittneben presented recent results they obtained in the domain of the project on Pervasive Wireless Access for 4G.

- Prof. Mikael Sternad, Uppsala University; The WINNER beyond 3G air-interface: A first outline

Prof. Sternad gave a very informative presentation on the results that they obtained concerning  the project on Wide Area Coverage and High Mobility Access Systems for 4G.

- Prof. Andreas Polydoros, University of Athens; Radio flexibility at the PHY layer

This presentation was related to the project on  the recent advances in the area of SDR.

- Prof. Hermann Rohling, Technical University Hamburg-Harburg; OFDM based systems and related multiple-access schemes

Prof. Rohling gave an excellent presentation on the potential of OFDM as a transmission and multiple-access technique for the next generation systems in comparison with the other candidates.

5.3 Tutorial/Review papers:

The list of tutorial/review papers is presented in chronolohical order:

- 5th MCM  and 1st COST 289 Workshop in Budapest, 7-9 July 2004:

Hiroyuki Atarashi, DoCoMo, Broadband Packet Wireless Access and Its Field Experiments.

Lajos Hanzo, University of Southampton, Recital on Multicarrier Communications: Space-Time Coded Versus Adaptive OFDM/MC-CDMA.

Armin Wittneben, ETH Zurich, Challenges in Pervasive Wireless Access

Joan Lluis Pijoan, Ramon Llull University, Software Radio: An Enabling Technology for Mobile Communications.

- 6th MCM in Barcelona, 28-29 October 2004:

Simone Morosi, University of Florence, Reconfigurable Antennas for Future Wireless Communications (Tutorial provided in the framework of NEXWAY, European Network of Excellence)

- 7th MCM in  Oberpfaffenhofen, Munich,7-8 March 2005:

Stefan Kaiser, DLR, Overview on MC-CDMA

- 8th MCM, Antalya, 6-8 July 2005:

A. Wittneben, ETH Zurich, Recent Theoretical and Experimental Results in Multiuser Zero Forcing Relaying

A. Polydoros, University of Athens, Radio flexibility at the PHY layer

H. Rohling, Technical University of Hamburg-Harburg,  OFDM Systems and Related Multiple Access Schemes

M. Sternad, Uppsala University, The WINNER Beyond 3G Radio Interface Concept

- 9th MCM, 3-4 November 2005 Madrid:

Álvaro Álvarez Vázquez, ACORDE, Ultrawideband Communications: state-of-the-art and research issues.

The tutorial was considered to be very useful and attracted a lot of interest among the participants. It provided useful information about this new and interesting area of research, which could not be covered by the Action because of the lack of sufficient number of scientists who are interested in this area.

- 10th MCM, 23-24 March 2006, Novi Sad:

Prof. Dr. Miroslav Lutovac, Prof. Dr. Dejan Tosic, University of Belgrade, Software for algebraic-symbolic and numerical processing in communication systems

The tutorial emphasized the advantages of symbolic processing in the analysis of telecommunication systems. The authors demonstrated the usefulness of their approach by some examples in various fields of electrical and electronics engineering.

Dr. Vesna Crnojević-Bengin,  University of Novi Sad, Metamaterials - Concept and Applications

This very interesting tutorial clearly showed the great potential that the use of metamaterials have in telecommunication systems.

### 5.4  Publications

The detailed list of publications is presented in Annex I and they are also posted in the web site of the Action:  http://cost289.ee.hacettepe.edu.tr

  - Web site

The web site is used to present the activities of the Action. The web site address is http://cost289.ee.hacettepe.edu.tr. The site is divided into two sections: an open section and a section restricted to the participants of the Action only. In the open section the following information is available: Status, MoU, Progress Reports, Chair, Participating Organisations, Meetings, Signatories and Related Links. The part accessible only through password is the List of Publications, which contains the working documents of the WGs and some electronic copies of the presentations.

  - Scientific and Technical Co-operation

Close scientific and technical cooperation is desired with other relevant COST Actions, scientific institutions, and research programmes, especially in the EU framework programmes.

During 6th MCM in Barcelona, 28-29 October 2004, the following lecture was offered by the EU Networks of Excellence NEXWAY:

Prof.Simone Morosi, University of Florence, Reconfigurable Antennas for Future Wireless Communications (Tutorial provided in the framework of NEXWAY, European Network of Excellence)

The following lectures, were presented during the 2nd COST 289 Workshop, 6-7 July 2005, Antalya, by Prof. Mikael Sternad, from European Network of Excellence WINNER. and by Dr. P. Fazekas from the NoE NEWCOM.

Prof. Mikael Sternad, Uppsala University; The WINNER beyond 3G air-interface: A first outline,  2nd COST 289 Workshop, 6-7 July 2005, Antalya, Turkey.

Dr. Peter Fazekas, Budapest University of Technology and Economics,  Network of Excellence in Wireless COMmunication (NEWCOM); structure, aim and achievements, 2nd COST 289 Workshop, 6-7 July 2005, Antalya, Turkey.

This presentation created an opportunity to discuss the relative advantages of NoE’s and COST Actions. The discussions led to a common understanding that combining the financial flexibility of NoE’s and the administrative flexibility of COST Actions could lead to an optimuml solution.

The following lectures were given by eminent scientists, who are not a member of the  Action,  during the 1st Workshop in Budapest, 7-9 July 2004.

Hiroyuki Atarashi, DoCoMo, Broadband Packet Wireless Access and Its Field Experiments.

Lajos Hanzo, University of Southampton, Recital on Multicarrier Communications: Space-Time Coded Versus Adaptive OFDM/MC-CDMA.

  - Transfer of results

The transfer of the research results is considered to be a key issue and a number of target groups have been identified, e.g., technology developers and providers, scientists, industry representatives and standardisation bodies as the key recipients of information. The actual dissemination is implemented based on the Action web site. Other means will include

- Electronic media in which the research results is/will be entered in the Internet for easy retrieval by researchers and other professionals.

- Specific contacts with already established agencies, initiatives and programmes.

- Traditional means such as reports, academic journals, conferences, seminars and workshops.

  - Contacts in the ERA

-

- ECONOMIC DIMENSION

Currently 30 institutions from 19 signatory countries participate the Action, with, on the average, 2 representatives per country. The evolution of the number of the signatories and the amounts reimbursed by COST is listed below:

The list of the expenses reimbursed  by COST-ESF:

Period March -June 30 2003:	 	 6 signatories		Inaugural MCM

Period  2003-2004:			16 signatories		39 715 Euro

Period  2004-2005:			18+1 signatories 	58 735 Euro

Period  2005-2006 :			19 signatories		65 805 Euro (provision.)

Period  200-April 2007 :

As also suggested in the MoU, the economic value of the research conducted by each country per year can be estimated from the following

| ½ engineer/researcher/secretary | 50 000 Euros |
| --- | --- |
| ½ technician | 30 000 Euros |
| 2 Ph.D. students | 70 000 Euros |
| Equipment and material costs | 30 000 Euros |
| Travel | 20 000 Euros |
| Total per signatory per year | 200 000 Euros |

The total cost per signatory will be about 200 000 Euros per year and 800 000 Euros in four years-duration of the Action. The total economic dimension over four years for 19 nations is predicted to be approximately 15.2 Million Euros.

- SELF EVALUATION

N/A

ANNEX I

COST 289 PUBLICATION LIST

I. PRESENTATIONS MADE IN MCMs and COST289 WORKSHOPS

- M. A. Dangl, W.G. Teich, J. Lindner and J. Egle, Joint Iterative Equalization, Demapping and Decoding with a  soft Interference Canceller

- D. Galda and H. Rohling, A Low Complexity Transmitter Structure for OFDM-FDMA Uplink Systems

- B. Chen and H. Rohling, Joint Layer Optimization for OFDM based Radio Systems

- L. G. Alonso Zarate, MAC/RLC Protocol Design in Heterogeneous Networks

- S. Bozay and M. Safak, Performance Analysis of Spatial Multiplexing and Maximal Ratio Combining Systems in the Presence of Polarization Diversity

- A. Svensson, A. Ahlen, A. Brunstrom, T. Ottosson and M. Sternad, An OFDM based System Proposal for 4G Downlinks

- M. Sternad, T. Ottosson, A. Ahlen and A. Svensson, Attaining both Coverage and High Spectral Efficiency with Adaptive OFDM Downlinks

- W. Wang, T. Ottosson, M. Sternad, A. Ahlen and A. Svensson, Impact of Multiuser Diversity and Channel Variability on Adaptive OFDM

- M. des Noes and D. Ktenas (presented by Sylvie Mayrargue), MC-CDMA vs DS-CDMA

- Stephan Sand, Two Dimensional Pilot Symbol Aided Channel Estimation for a Broadband MC-CDMA System with High Mobility

- A.J. Han Vinck, Pulse Position Access Codes

- Pavol Švac, Martin Piekov, CC-CDMA for B3G Wireless Communications : An overview and some open isuues

- Dušan Kocur, Jana Cízová, Stanislav Marchevsky, The Piece-Wise Linear Microstatistic Multi-User Receiver

- F. Chiti, R. Fantacci, G. Mennuti, and D. Tarchi, A Novel Admission Control Algorithm for UMTS System

- Arne Svensson, Joint project on ”Wide area coverage and high mobility access systems for 4G”

- F. Adelantado, O. Sallent, J. Pérez-Romero, R. Agustí, Traffic Hotspots in UMTS networks: influence on RRM strategies

- Simon Plass, Rank-Codes for OFDM

- Victor P.Gil Jimenez and Ana Garcia Armada, Bit-loading in Hybrid OFDM (H-OFDM)

- Stefan Wendt, A tapped Delay Line Model of Multipath Channel for CDMA Systems

- Sami Chtourou, Raphael Visoz, and Antoine O. Berthet, A Class of Low Complexity Iterative Equalizers for Space-Time BICM over MIMO Block Fading Multipath AWGN Channel

- Lorenzo Caponi, Francesco Chiti, Romano Fantacci, A Dynamic Rate Allocation Technique for Wireless Communication Systems

- Gabor Jeney, Janos Levendovszky, Sandor Imre and Laszlo Pap, Quadratic Optimization with Stochastic Recurrent Neural Networks

- Matilde Sanchez Fernandez, Ma del Pilar Cantarero Recio and Ana Garcia Armada, Study of MIMO channel capacity for IST METRA models

- Francesco Chiti, Cross Layer Protocols Design for Next Generation Wireless Systems

- Arne Svensson, Performance of Multiband OFDM In IEEE UWB Channel Models

- Henry Silalahi and Yonas Djamianto, UWB Based on Pulsed Multiband

- Sebastien de la Kethulle and Geir E. Øien, Energy-optimized coded modulation for short-range communications on Nakagami-m fading channels

- Daniele Tarchi, Cross-layer design for Multiple access techniques in wireless communications

- Vegard Hassel, Mohamed-Slim Alouini, David Gesbert and Geir E. Øien, Minimizing Feedback Load for Nested Scheduling Algorithms

- Stephan Sand, Iterative Channel Estimation for Broadband High Mobility MC-CDMA System

- Lars Wischhof and Hermann Rohling, Data Dissemination in Ad-Hoc Networks Based on Inter-vehicle Communication

- Romano Fantacci, Turbo Soft Combining Hybrid ARQ Techniques: Theory and Application to 3G Wireless Networks

- Stefan Berger and Armin Wittneben, Experimental Performance Evaluation of Multiuser Zero Forcing Relaying in Indoor Scenario

- Zsolt Polgar, Vasile Bota and Mihaly Varga, Correction Capabilities of the Reed-Solomon Codes Decoded with the Guruswami-Sudan Algorithm

- Vasile Bota, Mihaly Varga and Zsolt Polgar, Simulation Programs for the Evaluation of the LDPC-Coded Multicarrier Transmissions

- Vasile Bota, Zsolt Polgar and Mihaly Varga, Performances of LDPC-Coded OFDM Transmissions and Applications on Fixed and Mobile Radio Channels

- Victor P. Gil Jimenez and Ana Garcia Armada, Bit-Loaded H-OFDM Performance in WPAN Environments

- Hiroyuki Atarashi, Broadband Packet Wireless Access and Its Field Experiments (tutorial)

- Vasile Bota, Zsolt Polgar, Mihaly Varga, Performances of the LDPC-Coded Adaptive Modulation Schemes in Multi-Carrier Transmissions

- Francesco Chiti, Cross Layer Protocols Design for Next Generation Wireless Systems

- Sami Chtourou, Raphaël Visoz, Antoine O. Berthet, Whitened Matched Filter versus Channel Shortening for ST-BICM over MIMO ISI Channels

- Jana Cízová, Performance of the Microstatistic Multi-User Receiver in the Base-Band DS-CDMA Transmission System

- Andreas Zalonis, Ioannis Dagres and Andreas Polydoros, Attributes of Real Time Intelligence in Flexible Radios

- Simon Plass, Armin Dammann, Stefan Kaiser, On Modeling and Analysis of a Coded OFDMA Downlink in a Multi-Cell Environment

- Markus A. Dangl, Christian Sgraja, Jürgen Lindner, Turbo Equalization for ISI Channels Incorporating Estimation Error Statistics

- Xavier Revés, Antoni Gelonch, A Platform Abstraction Layer (P-Hal) for Software Radio Equipment

- Ingmar Hammerström and Armin Wittneben, Impact of Relay Gain Allocation on the Performance of Cooperative Diversity Networks

- Lajos Hanzo, Li-Liang Yang, Matthias Münster and Byoung-Jo Choi, Recital on Multicarrier Communications: Space-Time Coded Versus Adaptive OFDM/MC-CDMA (tutorial)

- Gábor Jeney and László Pap, Bit Loading Algorithms for Adaptive OFDM Wireless Systems

- Zoltán Németh and Sándor Imre, Interference Cancellation in MIMO Systems

- Gökhan Pay and Mehmet Safak, Array Relay

- Boris Rankov and Armin Wittneben, Distributed Spatial Multiplexing in Wireless Networks

- Daniele Tarchi, Cross-layer design for Multiple access techniques in wireless communications

- Armin Wittneben, Challenges in Pervasive Wireless Access (tutorial)

- Sinja Brandes, A New Technique for Sidelobe Suppression in OFDM Systems

- Francesco Chiti, Andrea De Cristofaro, Romano Fantacci, Daniele Tarchi, Giovanni Collodi, Gianni Giorgetti, Antonio Manes, Energy Efficient Routing Algorithms for Application to Agro-Food Wireless Sensor Networks

- Doris Y. Yacoub, Werner G. Teich and Jürgen Lindner, MC-Cyclic Antenna Frequency Spread: A Novel Space-Frequency Spreading for MIMO-OFDM

- Adão Silva, Downlink Strategies for MISO TDD MC-CDMA Systems

- Dragana Bajic and Cedomir Stefanovic, Short sequences and cross-bifix analysis

- H. Rohling; OFDM Systems and Related Multiple Access Schemes

- Sezginer, H.Sari; An Overview of Symbol Predistortion Techniques for PAPR Reduction in OFDM and OFDMA Systems

- M. Deumal, I. Gutierrez and J. L. Pijoan; PAPR Reduction in Orthogonal MC and MC-SS Systems

- V.Jimenez, A.G.Armada; Reducing the feedback information in OFDM-based Adaptive Modulation Systems for 4G

- C. Toker, S. Lambotharan; Sensitivity of the Orthogonalization Methods for QO-STBC to Feedback Errors in an OFDM Environment

- M. Sternad; The WINNER Beyond 3G Radio Interface Concept

- M. Varga, V. Bota, and Z. Polgar; User-Bin Allocation Methods for Adaptive-OFDM Downlinks of Mobile Transmissions

- D. Radovic; Effects of Channel on Multiuser CFO Estimation for Interleaved OFDMA Uplink

- M.E.Çelebi, S.Sahin, Ü.Aygölü; Space-time Block Code Selection for More Than Two Transmit Antennas

- K.A. Hamdi; On the Multiple-access Capability of a Shared Rayleigh Wireless Channel

- D.Kocur, J.Cízová, S.Marchevsky; Sub-optimum MSF-MUD for CDMA Systems

- A. Wittneben; Recent Theoretical and Experimental Results in Multiuser Zero Forcing Relaying

- J. Zhao, A. Wittneben; Cellular Relaying Networks: State of the Art and Open Issues

- I.Hammerström, J.Zhao, A.Wittneben; Temporal Fairness Enhanced Scheduling for Cooperative Relaying Networks in Low Mobility Fading Environments

- S.Berger, A.Wittneben; Cooperative Distributed Multiuser MMSE Relaying in Wireless Ad-Hoc Networks

- R. Fantacci, D. Tarchi, G.Izzo; A MAC Protocol for High-speed Multimedia WPANs

- A.J.H.Vinck; Coding for a Terrible Channel

- Z.Polgar, F.Ardelean, M.Varga, V.Bota; Performance Comparison of LDPC Codes Generated with Various Code-Construction Methods

- A. Zsiros, A. Fülöp, G.Jeney; Extending UTRAN Physical Layer with Coded Modulation Schemes

- A.Mengi, G.Bauch, A.J.H.Vinck; Space-Time Differentially Coded Orthogonal Matrix Modulation using QAM

- A. Polydoros; Radio Flexibility at the PHY Layer

- M.A.Dangl, J.Lindner; Turbo Equalization with Parametric Uncertainties: Comparison of SNR Estimation Algorithms

- H.Senol, H.A.Çirpan, E.Panayirci, M.Çevik; KL-Expansion Based Channel Estimator for Space-time/frequency Coded OFDM Systems with Transmitter Diversity

- S.Sand, R.Raulefs, A.Dammann; Iterative Channel Estimation for MIMO MC-CDMA

- V. Poulkov, G.Iliev; Channel Equalization for OFDM

- A. Wittneben, S. Berger, I. Hammerstroem, B. Rankov; Recent Theoretical and Experimental Results in Multiuser Zero Forcing Relaying

- A. Gameiro; Scheduling, Diversity and QoS Issues

- D.Yacoub, C.Schneider, S.Warzuegel, W.G.Teich, R.Thoma and, J.Lindner; Capacity of Measured MIMO Channels in Dependence of Array Element Spacing and Distance between Antennas

- H. Sari; Frequency-Domain Techniques in Wireless Communications: OFDM, Precoded OFDM and SCT/FDE

- E.Auger, B.Rankov, M.Kuhn and A.Wittneben; Time Domain Precoding for MIMO-OFDM Systems

- I. Hammerström; Power Allocations for Nonregenerative MIMO-OFDM Relay Links

- S.Plass; Spreading Codes for Radio Resource Management in MC-CDMA

- Á.Á.Vázquez; UWB Technologies

- D. Vukobratovic; Optimality Considerations of Short-Length LDPC Codes Construction

- V.Bota, M.Varga, Zs.Polgar; Convolutional vs. LDPC Coding for Coded Adaptive-QAM Modulations on Mobile Radio Channels

- R.Visoz, S.Chtourou and A.O.Berthet; Turbo-Equalization for Single-Carrier Transmission over MIMO Broadband Wireless Channel: Performance/Compexity Trade-Offs

- M.Lutovac and D.Tosic; Symbolic analysis and design of communication systems using computer algebra systems

- Vesna Crnojevic-Bengin; Metamaterials - Concept and Applications

- E.Aktas, J.Evans and S.Hanly; Distributed Base Station Processing in the Uplink of Cellular Networks

- S.Berger and A.Wittneben; Optimal Power Loading for Orthogonal Multiuser Relaying

- J.Bastos and A.Gameiro; Equalization Schemes for Extended Alamouti Codes in MC-CDMA Systems

- J.A.R.Cantero and M.J.F.García; A MIMO-OFDM based system with backward compatibility with IEEE802.16

- S.Plass; Increasing Transmit Diversity at the Cell Border with Smart Antennas

- F.Chiti, M.Ciabatti, G.Collodi, D.Di Palma, R.Fantacci and A.Manes; Energy Efficient MAC Protocols Design for Wireless Sensor Networks

- V.Bota, Z.Polgar and M.Varga; Performances of Convolutionally-Coded QAM Mapping Non-Coded Bits

- C. Vilella, J.L. Pijoan; Software Radio: An Enabling Technology for Mobile Communications

- P. Bergadà, C. Vilella; Sodio: A Software Radio platform for advanced HF communications

- I. Gutiérrez, J.L. Pijoan, M.Deumal, F. Bader; Adaptive procedure combining adaptive user grouping and bit-loading in a GO-MC-CDMA

II. JOURNAL PAPERS, BOOK CHAPTERS AND BOOKS:

  - Sorour Falahati, Arne Svensson, Torbjörn Ekman, Mikael Sternad, Adaptive Modulation Systems for Predicted Wireless Channels. IEEE Trans. Communications, 52(2):307-316, Feb 2004.

  - Y Lee, S McLaughlin and E Al-Susa, “Pre-Selection Tentative Decision Device-Based SIR Estimator for a TD-CDMA System”, IEICE Letter, Trans. Communications, Vol. E87-B, No 10, Oct. 2004.

  - E Al-Susa, Y Lee and S McLaughlin, “Channel-Adaptive Sectored Multicarrier Packet Based Systems” IEE Electronics Letters, Volume 40, Issue 18, Page(s): 1194-1196, Sept. 2004.

  - E Al-Susa, D G M Cruickshank, Y LEE, S McLaughlin, "Preselection-based iterative multiuser detector for DS-CDMA systems", IEE Electronics Letters, Volume 40, Issue 9, Page(s): 546- 548, Apr. 2004.

- E. Al-Susa, D G M Cruickshank and S McLaughlin, “Practical algorithm for adaptive subcarrier-hopping multicarrier multiple access transmission”, Electronics Letters, Volume: 39 Issue: 21, Page(s) 1544 -1546, Oct. 2003.

- Khairi A. Hamdi, Exact Error Probabilities for DPSK Systems with Asynchronous Cochannel Interference in Rayleigh Fading Channels, IEEE Trans. Vehicular Technology, vol. 52, No. 6, Nov. 2003, pp. 1621 –1626.

- Khairi A. Hamdi and I. Henning, On the Channel Spacing of Multichannel ALOHA,
IEEE Communication Letters, vol. 7, No. 10, Oct. 2003,  pp. 469- 471.

- Khairi A. Hamdi, Hit Statistics in FH-CDMA Unslotted Packet Networks, IEEE Trans. Information Theory, vol. 49, no. 5, May 2003,  pp. 1363-1368

- R. Fantacci, F. Chiti, D. Marabissi, G. Mennuti, S. Morosi, and D. Tarchi, "Perspectives for present and future CDMA-based communications systems," IEEE Commun. Mag., vol. 43, no. 2, pp. 95-100, Feb. 2005

- R. Fantacci, A. Ferri, and D. Tarchi, “A Medium Access Control Protocol for CDMA Ad-Hoc Networks,” Electronics Letters, vol. 4, no. 18, pp. 1131-1132, Sep. 2004.

- R. Fantacci, L. Mancini, M. Marini, and D. Tarchi, “A neural network-based blind multiuser receiver for DS-CDMA communication systems,” Wireless Personal Communications, vol. 27, no. 3, pp. 195–213, 2003

- R. Fantacci, M. Forti, M. Marini, D. Tarchi, and G. Vannuccini, “A neural network for constrained optimization with applications to CDMA communication systems,” IEEE Trans. Circuits Systems, II, vol. 50, no. 8, Aug. 2003.

- S. Imre, F. Balázs:  Quantum Computing and Communications – An Engineering Approach, Published by John Wiley and Sons Ltd, The Atrium, Southern Gate, Chichester, West Sussex PO19 8SQ, England, 2005, ISBN 0-470-86902-X, 283 pages.

- S. Imre et al: “Chapter 5: Network Architectures and Functions”, Software Defined Radio: Architectures, Systems and Functions, Edited by Marcus Dillinger, Published by John Wiley and Sons Ltd, The Atrium, Southern Gate, Chichester, West Sussex PO19 8SQ, England, 2003, ISBN 0-470-85164-3, pp. 95-141.

- Gy. Rábai, S. Imre: “Chapter 13: Reconfiguration of Network Elements”, Software Defined Radio: Architectures, Systems and Functions, Edited by Marcus Dillinger, Published by John Wiley and Sons Ltd, The Atrium, Southern Gate, Chichester, West Sussex PO19 8SQ, England, 2003, ISBN 0-470-85164-3, pp. 355-371.

- Z. Németh, S. Imre, F. Balázs, Link Adaptation in MIMO Systems, Telecommunications, Vol. LVIII, No.6., Selected Papers, 2003. pp. 14-21.

- Víctor P. Gil Jiménez, M.J. Fernández-Getino. Francisco J. González Serrano, Ana García Armada, Design and implementation of synchronization and AGC for OFDM-based WLAN receivers, IEEE Trans. Consumer Electronics Vol. 50, pp: 1016- 1025, Nov. 2004.

- Ana García Armada, Francisco Javier Ramos López, Francisco Javier González Serrano, Wireless Communications Systems, Wiley Encyclopedia of RF and Microwave Engineering, John Wiley & Sons,  ISBN: 0-471-27053-9, 2004.

- Manuel J.L. Rojo-Álvarez, M. Martínez-Ramón, A.R. Figueiras-Vidal, A. García-Armada, A. Artés-Rodríguez, A robust support vector algorithm for nonparametric spectral analysis, IEEE Signal Processing Letters, vol.: 10, pp: 320 – 323, Nov 2003.

- Jianhua Zhang, Hermann Rohling, Ping Zhang: Analysis of ICI Cancellation Scheme in OFDM Systems with Phase Noise. IEEE Tran. Broadcast, Vol. 50, No. 2, pp. 97 –106, June 2004.

- H. Rohling and D. Galda, OFDM Transmission Technique: A Strong Candidate for Next-Generation Mobile Communications, Radio Science Bulletin, no. 310, September 2004, pp.47-58.

- Luis Alonso, Ramon Agustí, Automatic Rate Adaptation and Energy–saving Mechanisms based on Cross-Layer Information for Packet-Switched Data Networks, IEEE Communications Magazine, Vol. 43, No. 3, pp. S15-S20, March, 2004.

- Xavier Revés, Antoni Gelonch, Vuk Marojevic, Ramon Ferrús, Software Radios: unifying the reconfiguration process over heterogeneous platforms, accepted for publication in EURASIP JASP (Journal on Applied Signal Processing), February, 2005.

- Yuan Luo, Wende Chen, and A. J. Han Vinck,  The Determination of the Chain Good Weight Hierarchies with High Dimension, SIAM Journal on Discrete Mathematics, Volume 17, Number 2, Pages 196 - 209, 2004.

- Jurgen Haering and  A J Han Vinck, Coding and Signal Space Diversity for a Class of Fading and Impulsive  Noise Channels,  IEEE Trans. Information Theory,  May 2004 , pp. 887-895.

- C.P.M.J. Baggen, A.J. Han Vinck and A. Nowbakht-Irani, Communication and Modulation, Information Theory in the Benelux, An Overview of WIC Symposia, 1980-2003, ISBN 90-71048-19-, pp. 117-130.

- Fang-Wei Fu, A.J. Han Vinck, Victor Wei, and Raymond Yeung, On the Capacity of Write-Unidirectional Memories with Nonperiodic Codes, IEEE Trans. Information Theory, April 2004, pp. 649-656,

- Jürgen Häring and A.J. Han Vinck, Iterative Decoding of Codes over Complex Numbers for Impulsive Noise Channels, IEEE Trans. Information theory, May 2003, pp. 1251-1260.

- Y. Luo, F. Fu, A. J. Han Vinck and W. Chen, On Constant Composition Codes over Zq, IEEE Trans. Information Theory, November 2003, pp. 3010-3016.

- V. B. Balakirsky and A.J. Han Vinck, On the Performance of Permutation Codes for Multi-User Communciations, Problems of Information Transmission, No. 3., 2003, pp. 239-254. (ISSN: 0032-9460).

- Y. Luo, A.J. Han Vinck, W. Chen and F. Fu, A new kind of geometric structures determining the chain good weight hierarchies, Discrete Mathematics, vol.260C, pp.101-117, Jan. 2003.

- Kocur, D.-Hendel, I.: PWL vs. Conventional Microstatistic Digital Filters. Acta Electrotechnica et Informatica, vol.5, No.1, 2005, pp. 22-26.

- Kocur, D. - Čížová, J. - Marchevský, S.: Microstatistic Multi – User Detection Receiver. Journal of Advanced Computational Intelligence and Intelligent Informatics (JACI3), Japan, vol. 8., No.5, 2004, pp. 482-487.

- Longauer, L.- Marchevský, S.- Kocur, D.: BAMUD Features Demonstration by System View. Radioengineering, vol.13, No. 3, 2004, pp.47-52.

- Kocur, D.-Zetík, R.: Volterra Filters: A Promising Tool for Wideband and Narrowband Interference Suppression in DS-SS Communication Systems. Iranian Journal of Electrical and Computer Engineering (IJECE), vol 2., No. 1., 2003, pp. 1-8.

- Kocur, D.-Čížová, J.: Multi-user Detection Techniques for CDMA: A Review of Basic Principles, Acta Electrotechnica et Informatica, vol.3, No.1, 2003, Slovakia, pp. 28-35

- Kocur, D.-Čížová, J.-Marchevský, S.: Nonlinear Microstatistic Multi-User Receiver. Acta Electrotechnica et Informatica, vol.3., No.3, 2003, Slovakia, pp. 10-15.

- R. Raulefs, A. Dammann, S. Sand, S. Kaiser, and G. Auer, Rotated Walsh-Hadamard Spreading with Robust Channel Estimation for a Coded MC-CDMA System, EURASIP Journal on Wireless Communications and Networking - Special Issue on Innovative Signal Transmission and Detection Techniques for Next Generation Cellular CDMA Systems, Vol. 2004, No. 1, August 2004, pp. 74-83.

- G. Auer, S. Sand, A. Dammann, and S. Kaiser, Analysis of Cellular Interference for MC-CDMA and its Impact on Channel Estimation, European Transactions on Telecommunications, Vol. 15, May/June 2004, pp. 173-184.

- Y. Luo, C. Mitrpant, A.J. Han Vinck, K. Chen., "Some New characters on the Wire-Tap Channel of Type II", IEEE Trans. Information Theory, March 2005, pp. 1222-1228, ISSN 0018-9448.

- H.C. Ferreira, A.J. Han Vinck, T.G. Swart and I. de Beer, "Permutation Trellis Codes", IEEE Trans. Communications, pp. 1782-1789, November 2005

- D. Bajic, “Search, Sequences, Synchronization and States: a Different Approach”, a monograph (in English), University of Novi Sad, 2006 (reviewers: Dr. W. Teich, University of Ulm, Dr. T. Willink, Communications Research Centre, Ottawa)

- D. Bajic, D. Drajic, “Statistical Analysis of Digital Signals and Systems”, Chapter 7 in Coding and Signal Processing for Magnetic Recording Systems, Bane Vasic and Erozan Kurtas (Editors), CRC Press LLC, 2005, ISBN 0-8493-1524-7

- D. Bajic, J. Stojanovic, “Frame-alignment procedures for STM-1 frame”, IEE Proceedings, Communications, Vol. 150, No. 1, 2003, pp. 37-44.

- Ana García Armada, “SNR Gap Approximation for M-PSK-based Bit Loading”, IEEE Trans. Wireless Communications, vol 5, n 1. pp. 57 – 60,. January 2006.

- M.J. Fernández-Getino García, O. Edfors, J.M. Páez-Borrallo, “Peak power reduction for OFDM systems with Orthogonal Pilot Sequences”, IEEE Trans. Wireless Communications, vol 5, n 1. pp. 47 - 51. January, 2006.

- M. Julia Fernández-Getino García, J. Luis Rojo Álvarez, Víctor P. Gil Jiménez, Felipe Alonso Atienza, Ana García Armada, “A complex support vector machine approach to OFDM coherent demodulation”, Book Title: “Kernel Methods in Bioengineering, Signal and Image Processing. ISBN 1-59904-042-5. 2006.

- S. Plass, "Hybrid partitioned cellular downlink structure for MC-CDMA and OFDMA", IEE Electronics Letters, vol. 42, no. 4, pp. 226-228, Feb., 2006.

- Rydström, M., Urruela, A., Ström, E. G. et al., "Autonomous Positioning Techniques Based on Cramer-Rao Lower Bound Analysis", EURASIP Journal on Applied Signal Processing, 2006, pp.1-10. ISBN/ISSN: 1110-8657 CPL 19445

- Mielczarek, B., Svensson, A., "Modeling Fading Channel-Estimation Errors in Pilot-Symbol-Assisted Systems, With Application to Turbo Codes", IEEE Trans.  Communications. 53 (11) pp. 1822-1832. CPL 11976, 2005

III. CONFERENCE PAPERS:

- W. Zhang and J. Lindner, Modeling of OFDM-based Systems with Frequency Offsets and Frequency Selective Fading Channels, in Proc. IEEE 61st Semiannual Vehicular Technology

Conference (VTC), Stockholm, Sweden, May/June 2005.

- W. Zhang and J. Lindner,  MC-CDMA in the Uplink: A transmission Scheme whichTolerates Frequency Offsets and Frequency Selective Fading Channels, in Proc. IEEE 15th Int. Symposium on Personal, Indoor and Mobile Radio Comms., Barcelona, Spain, Sept. 2004.

- D. Yacoub, E. Msechu, W. G. Teich, J. Lindner, Effect of Spreading in an Outdoor MIMO-OFDM System, in Proc. 9th International OFDM-Workshop, Dresden, Germany, Sept. 2004.

- M. A. Dangl, C. Sgraja, J. Lindner, On Unbiased Minimum Variance Equalization with Channel Estimation Uncertainty, Conference on Information Sciences and Systems, pp. 609-614, Princeton (NJ), USA, March 2004.

- W. Zhang, M. A. Dangl, and J. Lindner, Performance Analysis of the Downlink and Uplink of MC-CDMA with Carrier Frequency Offset", 4th International Workshop on Multi-Carrier Spread Spectrum (MC-SS), Oberpfaffenhofen, Germany, September 2003, published in: Khaled Fazel and Stefan Kaiser (eds.), "Multi-Carrier Spread-Spectrum", pp. 259-267, Kluwer Academic Publishers, Netherlands, January 2004.

  - D. Yacoub, M. A. Dangl, U. Marxmeier, W. Teich, J. Lindner, Comparison of Iterative Detection Schemes for MIMO Systems with Spreading Based on OFDM, 4th International Workshop on Multi-Carrier Spread Spectrum (MC-SS), Oberpfaffenhofen, Germany, September 2003, published in: Khaled Fazel and Stefan Kaiser (eds.), "Multi-Carrier Spread-Spectrum", pp. 101-111, Kluwer Academic Publishers, Netherlands,  January 2004.

- M. A. Dangl, C. Sgraja, W. G. Teich, J. Lindner, J. Egle, Convergence Behavior of Iterative Equalization and Decoding Schemes with Memory, in Proc. IEEE Global Telecommunications Conf., vol. 3, pp. 1710-1715, San Francisco (CA), USA, Dec. 2003.

- M. A. Dangl, W. G. Teich, J. Lindner, J. Egle, Joint Iterative Equalization, Demapping, and Decoding with a Soft Interference Canceler, in Proc. 7th International Symposium on Communication Theory and Applications, pp. 36-41, Ambleside, England, July 2003.

- M. A. Dangl, D. Yacoub, U. Marxmeier, W. G. Teich, J. Lindner, Performance of Joint Detection Techniques for Coded MIMO-OFDM and MIMO-MC-CDM, in Proc. COST 273 Workshop on Broadband Wireless Local Access, pp. 17/1-17/6, Paris, France, May 2003.

- Mats Rydström, Arash T. Toyserkani, Erik G. Ström, Arne Svensson, Towards a Wireless Network for Traffic Safety Applications. Proceedings RadioVetenskap och Kommunikation, Linköping, Sweden, June 2005.

  - Mats Rydström, Andreu Urruela, Erik Ström, Arne Svensson, Practical Automotive Applications of Cramer-Rao Bound Analysis. Proceedings of the IEEE Intelligent Vehicles

symposium, Las Vegas, Nevada, USA, June 2005.

  - Mats Rydström, Andreu Urruela, Erik G. Ström, Arne Svensson, A Low Complexity Algorithm for Sensor Localization. Proceedings of the 11th European Wireless Conference, Nicosia, Cyprus, April 2005.

  - Mats Rydström, Andreu Urruela, Erik Ström, Arne Svensson, Low Complexity Tracking for Ad-hoc Automotive Sensor Networks. Proceedings The First IEEE Int. Conference on Sensor and Ad hoc Communications and Networks, Santa Clara, California, USA, Oct 2004.

  - Florent Munier, Thomas Eriksson, Arne Svensson, Receiver Algorithms for OFDM systems in Phase Noise and AWGN. Proceedings  IEEE International Symposium on Personal, Indoor, and Mobile Radio Communications, Barcelona, Spain, Sep 2004.

  - Matts-Ola Wessman, Arne Svensson, Erik Agrell, Frequency Diversity Performance of Coded Multiband-OFDM Systems on IEEE UWB Channels. Proceedings IEEE Vehicular Technology Conference Fall, Los Angeles, California, USA, Sep 2004.

  - Matts-Ola Wessman, Arne Svensson, Comparison between DS-UWB, Multiband UWB and Multiband OFDM on IEEE UWB Channels. Proceedings Nordic Radio Symposium and Finnish Wireless Communications Workshop, Oulu, Finland, Aug 2004.

  - Mats Rydström, Andreu Urruela, Erik Ström, Arne Svensson, Node-aided Locationing and Tracking for Low Cost Ad-hoc Automotive Sensor Networks. Proceedings 11th Wireless World Research Forum Workshop, Oslo, Norway, June 2004.

  - Arne Svensson, Mats Rydström. A Communication Network for Safe Traffic and Efficient Transportation. Proc. 11th Wireless World Research Forum Workshop, Oslo, Norway, June 2004.

  - Sorour Falahati, Arne Svensson, Torbjörn Ekman, Mikael Sternad, Adaptive Modulation Systems for Predicted Wireless Channels. Proceedings Global Telecommunications Conference, San Francisco, California, USA, Dec 2003.

  - Florent Munier, Eric Alpman, Thomas Eriksson, Arne Svensson, Herbert Zirath, Estimation of Phase Noise for QPSK Modulation over AWGN Channels. Proceedings Gigahertz 2003 Symposium, Linköping, Sweden, Nov 2003.

  - Ali Behravan, Thomas Eriksson, Arne Svensson, Herbert Zirath, Adaptive predistorter design for nonlinear high power amplifiers. Prodeedings of GHz 2003 Symposium, Linköping, Sweden, Nov 2003.

  - Sorour Falahati, Mei Hong, Arne Svensson, Mikael Sternad, Adaptive Trellis Coded Modulation over Predicted Flat Fading Channels. Proceedings Vehicular Technology Conference Fall, Orlando, Florida, USA, Oct 2003.

  - Mikael Sternad, Tony Ottosson, Anders Ahlén, Arne Svensson, Attaining both Coverage and High Spectral Efficiency with Adaptive OFDMA Downlinks. Proceedings Vehicular Technology Conference Fall, Orlando, Florida, USA, Oct 2003.

  - Wei Wang, Tony Ottosson, Mikael Sternad, Anders Ahlén, Arne Svensson. Impact of Multiuser Diversity and Channel Variability on Adaptive OFDM. Proceedings Vehicular Technology Conference Fall, Orlando, Florida, USA, Oct 2003.

  - Bartosz Mielczarek, Matts-Ola Wessman, Arne Svensson, Performance of Coherent UWB RAKE Receivers with Channel Estimators. Proceedings IEEE Vehicular Technology Conference Fall, Orlando, Florida, USA, Oct 2003.

  - Tommy Svensson, Arne Svensson, Empirical Model for Spectrally Efficient Continuous Phase Modulation. Proceedings Vehicular Technology Conference Fall, Orlando, Florida, USA, Oct 2003. .

  - Arne Svensson, Anders Ahlén, Anna Brunstrom, Tony Ottosson, Mikael Sternad, An OFDM based System Proposal for 4G Downlinks. Proceedings International Workshop on Multi-Carrier and Spread Spectrum for Future Generation Wireless Systems, Oberpfaffenhofen, Germany, Sep. 2003. Invited paper.

  - Bartosz Mielczarek, Matts-Ola Wessman, Arne Svensson, Performance of Coherent UWB Rake Receivers using Different Channel Estimators. Proceedings IEEE Internation Workshop on Ultra Wideband Systems - UWB Cluster Day, Oulu, Finland, June 2003.

  - Maxime Flament, Arne Svensson, Virtual Cellular Networks for 60 GHz Wireless Infrastructure. Proc. of International Conference on Communications, Anchorage, Alaska, USA, May 2003.

  - Tommy Svensson, Arne Svensson, Constrained Envelope Continuous Phase Modulation. Proceedings Vehicular Technology Conference Spring, Jeju, Korea, April 2003.

  - M. Luna Rivera and E Al-Susa, “Increasing capacity in a FEC Coded DS-CDMA System over Frequency-Selective Channels”, IEEE International Symposium on Personal, Indoor and Mobile Radio Communications, PIMRC 04, Barcelona, Spain, Sept. 2004

- E Al-Susa, D Cruickshank and S McLaughlin, “Priority Swapping Sub-carrier and User Allocation Technique for Adaptive Multi-carrier Based Systems”, (plenary paper) Fourth international Workshop on Multi-Carrier Spread-Spectrum, MCSS 03, and Related Topics, Sept. 2003.

- M Tabulo, D Laurenson, S McLaughlin and E Al-Susa, “A Linear Programming Algorithm for a Grouped MC-CDMA System”, Proceedings of IEEE Vehicular Technology Conference, VCT 03-Spring, Orlando, USA, Oct. 2003.

- P M Grant, J S Thompson, C C Chong, Y Lee, S K Yong, E Alsusa, “Mobile communications activities in wireless access”, IEEE Personal on Indoor and Mobile Radio Communications

Conference, PIMRC 03. Volume: 3, Page(s): 2650 – 2654, Sept. 2003.

  - J M Luna Rivera, E Al-Susa, M C E Stevens Navarro, J A Elías, M C Ulises Pineda, M C J G Pérez, “Orthogonal DS-CDMA System In Multipath Fading Channels”, IEEE ROC&C Conference, Ponencias, Nov. 2003.

  - P Sedtheetorn and K A Hamdi, Throughput Analysis of Multirate VSG CDMA Wireless Packet Communication Systems in Rayleigh Fading Environment, IEEE WCNC’05 (IEEE Conference on Wireless Communication and Networking,) New Orleans, USA, 13-17 March 2005.

  - Khairi A. Hamdi, On the Reliability of Outage Models, IEEE VTC’04-Fall (IEEE Conference on Vehicular Technology) Los Angeles, USA, 26-29 Sept. 2004.

  - Khairi A. Hamdi, On the Multiple-Access Capability of the Bi-Phase Modulated TH-CDMA Impulse Radio Networks, IEEE VTC’04-Fall (IEEE Conference on Vehicular Technology) Los Angeles, USA, 26-29 Sept. 2004.

  - Khairi A. Hamdi and Omar Bamahdi, A New Adaptive Frequency Hopping Technique
IEEE VTC’04-Fall (IEEE Conference on Vehicular Technology) Los Angeles, USA, 26-29 Sept. 2004.

  - Khairi A. Hamdi, On the Spectral Efficiency of Shared Wireless Channels, IEEE GLOBECOM’03 (IEEE Global Communication Conf.), San Francisco USA, 1– 5 Dec. 2003.

  - Khairi A. Hamdi and I. Henning , On the Performance of FH/BFSK Multiple Access Communication in Systems with Overlapped Hopping Channels, IEEE GLOBECOM’03 (IEEE Global Communication Conference), San Francisco USA, 1 – 5 Dec. 2003

  - R. Fantacci, A. Ferri, and D. Tarchi, "A MAC technique for CDMA based ad-hoc networks," in Proc. of IEEE WCNC 2005, Mar. 2005, New Orleans, LA, USA.

  - M. Bandinelli, F. Chiti, R. Fantacci, D. Tarchi, G. Vannuccini, "A link adaptation strategy for QoS support in IEEE 802.11e-based WLANs," in Proc. of IEEE WCNC 2005, Mar. 2005, New Orleans, LA, USA.

  - R. Fantacci, R. Gubellini, T. Pecorella, and D. Tarchi, “DiffServ on-board satellite switching based on cellular neural networks,” in Proc. of IEEE ICC 2004, June 2004, Paris, France.

  - R. Fantacci, A. Ferri, and D. Tarchi, “A novel MAC technique for ad-hoc CDMA networks,” in Proc. of  IWWAN’04, June 2004, Oulu, Finland.

  - R. Fantacci, M. Forti, M. Marini, A. Rabbini, and D. Tarchi, "Proposal of an Advanced MMSE Multiuser Receiver for a DS-CDMA Environment using Neural Networks", in Proc. of IEEE Globecom 2003, Dec. 2003, San Francisco, CA, USA

  - R. Fantacci, R. Gubellini, T. Pecorella,, D. Tarchi, and F. Chiti, "An Efficient DiffServ Switch for Satellite Communication Systems based on Cellular Neural Networks", in Proc. of VTC2003 Fall, Oct. 2003, Lake Buena Vista, FL, USA.

  - F. Chiti, R. Fantacci, G. Mennuti, and D. Tarchi, "An Efficient Soft Admission Control Technique for Wireless Communications, in Proc. of VTC2003 Fall, Oct. 2003, Lake Buena Vista, FL, USA.

  - R. Fantacci, R. Gubellini, T. Pecorella, and D. Tarchi, “A cellular neural networks based di serv switch for satellite communication systems,” in Proc. of ASMS 2003, July 2003, ESRIN, Frascati, Italy.

  - R. Fantacci, R. Gubellini, T. Pecorella, and D. Tarchi, “Satellite switching with neural network techniques for di serv policy,” in Proc. of Int. Workshop of COST Actions 272 and 280, May 2003, ESTEC, Noordwijk, The Netherlands.

  - F. Chiti, R. Fantacci, G. Mennuti, and D. Tarchi, “Dynamic SIR based admission control algorithm for 3G wireless networks,” in Proceedings of IEEE ICC 2003, May 2003, Anchorage, AK, USA.

  - R. Fantacci, D. Marabissi, L. Panichi and D. Tarchi, “Performance evaluation of an efficient fixed microwave communication system to be added to an operating UMTS network,”  in Proc. of IEEE ICC 2003, May 2003, Anchorage, AK, USA.

  - S. Imre, K. Hankó, P. Petrás, R. Tancsics: ”Efficient Call Admission Control Method for 3G/4G WCDMA Networks”, 7th International Conference on Telecommunications  CONTEL2003, June 13-15, 2003, Zagreb, Croatia, Vol. 1., Published by CIP Zagreb, ISBN 953-184-055-5, pp.293-300.

  - S. Imre, L. Pap, F. Balázs, J. Cz. Horváth, R. Schulcz, S. Szabó: „The SONG (SOlutions for Next Generation mobile systems) Project”, Personal Wireless Communications, PWC 2003, September 23-25, 2003, Venice, Italy, Published at Springer, Lectures Notes in Computer Sciences, Vol. 2775, ISBN 3-540-20123-8, pp. 207-212.

  - S. Imre: ” Dynamically Optimised Chernoff bound Based CAC for 3G/4G WCDMA Systems”, 11th Microcoll Conference, September 10-11, 2003, Budapest, Hungary, ISBN 963-212-166-X,  pp.27-30.

  - Z. Németh, S. Imre: “Source separation in MIMO Systems”, 8th International OFDM-Workshop, InOWo’03, 24.-25. September 2003, Hamburg, Germany, pp.207-211.

  - Z. Németh, S. Imre: “Channel Equalization and Source Separation in MIMO Systems”, 2003 Polish-Czech-Hungarian Workshop on Circuit Theory, Signal Processing, and Telecommunication Networks, September 11-13, 2003, Prague, Czech Republic, ISBN 80-01-02825-9, pp.109-112.

  - S. Imre, P. Petrás, R. Táncsics: “Efficiency Validation of 3G/4G WCDMA Air Interface Call Admission Control in OMNeT++ Environment”, SoftCOM2003, October 07-10, 2003, Split, Dubrovnik (Croatia), Ancona, Venice (Italy), Published at FESB-Split, ISBN 953-6114-64-X, pp. 852-858.

  - S. Imre: “Quantum Computing Based Feedback Channel Coding for Medium Access Control”, SoftCOM2003, October 07-10, 2003, Split, Dubrovnik (Croatia), Ancona, Venice (Italy), Published at FESB-Split, ISBN 953-6114-44-5, pp.516-520.

  - G. Hamar, Z. Kotormán, G. Rábai, S. Imre: “Resource Allocation in a Software-Radio Environment”, 4nd Karlsruhe Workshop on Software Radios, WSR2004, March 17-18., 2004, Karlsruhe, Germany, pp. 1-7.

  - M. Erdei, S. Imre, G. Rábai, K. Sója: “Analyzing Software Configurations on Reconfigurable Hardware Devices”, 4nd Karlsruhe Workshop on Software Radios, WSR2004, March 17-18., 2004, Karlsruhe, Germany,  pp. 129-132.

  - S. Szabo, S. Imre, A. Burulitisz: “On the Accuracy of Mobility Modelling in Wireless Networks”, IEEE International Conference on Communications, ICC2004, 20-24 June, 2004, Paris, France, ISBN 0-7803-8534-9.

  - T. Radvánszki, B. Benkovics. Cs. I. Szabó, S. Imre: “Virtual Transmission Based MAC Protocol in Wireless Networks”, EUNICE 2004 (European Network of Universities and Companies in Information and Communication Engineering) 10th Open European Summer School on Advances in Fixed and Mobile Networks, June 14-16, 2004, Tampere, Finland, ISBN ISBN 952-15-1187-7, pp. 26-31.

  - R. Simon, S. Imre: Resource Management in Software Radio Environment, 46th International Symposium ELMAR-2004 focused on Navigation, Multimedia and Marine, 16-18 June 2004, Zadar, Croatia, ISSN 1334-2630, IEEE Catalog Number 04EX815, pp 443-448.

  - T. Radvánszky, B. Benkovics, S. Imre: „Virtual Transmission Based MAC Protocol for Wireless Access”, 4th Conference on Communication Systems, Networks and Digital Signal Processing, CSNDSP2004, 20-22 July 2004, Newcastle upon Tyne, UK, pp 445-448.

  - S. Imre, F. Balázs, Z. Németh, M. Katona, A. Mráz: “Dynamic Call Admission Control” Inter-University Reseach Center for Telecommunications and Informatics, Project Report, June 2004. pp. 48-53.

  - S. Imre: “Dynamic CAC for 3G/4G WCDMA Systems”, SoftCOM2004, October 10-13, 2004, Split, Dubrovnik (Croatia), Ancona, Bari (Italy), Published at FESB-Split, ISBN 953-6114-69-0, pp. 424-428.

  - F. Balazs, S. Imre: “Modified Radial Basis Network Based Blind Channel Estimation”, SoftCOM2004, October 10-13, 2004, Split, Dubrovnik (Croatia), Ancona, Bari (Italy), Published at FESB-Split, ISBN 953-6114-69-0, pp. 449-452.

  - Víctor P. Gil Jiménez, Ana García Armada, Bit-Loaded H-OFDM to increase capacity in WLAN/WPAN, IEEE Vehicular Technology Conference (VTC) Fall, Los Angeles, USA, September 2004.

  - Ana García Armada, Beatriz Bardón Rodríguez, Matilde Sánchez Fernández, Modelling, performance analysis and design of WPAN systems, Mobile eConference 2004 Online, August-October 2004.

  - Matilde Sánchez Fernández, Ana García Armada, Turbo Coding and Iterative decoding, Tipo de participación: autor, Online, August-October 2004.

  - Víctor P. Gil Jiménez, Mª Julia Fernández-Getino, Ana García Armada, H-OFDM channel estimation, synchronization and performance evaluation for Wireless Personal Communications, IST Mobile & Wireless Communications Summit, Lyon, France, June 2004.

  - A. García Armada, Beatriz Bardón, M. P. Sánchez, Modelling, performance analysis and design of WPAN systems, Workshop on Adaptive Global Net: Visions and Beyond, Lyon, France, June 2004.

  - A. García Armada, Analysis of phase noise effects in multi-user OFDM, IEEE Intl. Symposium on Control, Communications and Signal Processing (ISCCSP’04), Hammamet, Tunis, March 2004.

  - Ana García-Armada, Juan Ramón de Torre, Víctor P. Gil Jiménez, M. Julia Fernández-Getino García, Evaluation of different spreading sequences for MC-CDMA in WLAN environments, Multi-Carrier Spread-Spectrum, ed. by K. Fazel and S. Kaiser, Kluwer Academic Publishers, 2003. ISBN 1-4020-1837-1, Oberpfaffenhofen, Germany, September 2003.

  - B. Bardón, M. P. Sánchez, A. García Armada, BER evaluation of Reed Solomon codes using Importance Sampling, Intl. Conference on Applied Simulation and modelling, Marbella, Spain, September 2003.

  - B. Bardón, V. Gil, M.M. Olmedilla, M. P. Sánchez, A. García-Armada, Discrete channel simulation of general packet radio service (GPRS), First International Working Conference on Performance Modelling and Evaluation of Heterogeneous Networks (HET-NETs '03), Ilkley, UK, July 2003.

  - Ana García Armada, Víctor P. Gil Jiménez, Mª Julia Fernández-Getino García, José L. García, H-OFDM design for Wireless Personal Area Communications, IST Mobile & Wireless Communications Summit, Aveiro, Portugal, June 2003.

  - Hermann Rohling: A low complexity transmitter structure for OFDM - FDMA Transmitter Structure 10th International Scientific Conference, Tuapse, Russia, October 2004.

  - Hermann Rohling: OFDM Transmission Technique: A strong candidate for the next generation of mobile communications. URSI 2004, Barcelona, September 2004.

  - Hermann Rohling, Nico Tönder, Marc Reinert, Lars Ohliger: Flexible OFDM Demonstrator with a single FPGA Implementation. Proceedings 9th international OFDM workshop, Dresden, Germany, September 2004.

  - Ting Chen, Hermann Rohling, Elena Costa: Signal and Interference Measurements for Resource Allocation Schemes in OFDM-based Cellular Networks. Proceedings. 9th international OFDM workshop, Dresden, Germany, September 2004.

  - Jianjun Ran, Alexander Vanaev, Rainer Grünheid, Hermann Rohling: A MIMO-OFDM System with Long-term Channel State Information at Transmitter. Proceedings 9th international OFDM workshop, Dresden, Germany, September 2004.

  - Sonom Olonbayar, Hermann Rohling: Comparison between Coded OFDM-TDMA and OFDM-CDMA Systems. Proceedings 9th international OFDM workshop, Dresden, Germany, September 2004.

  - Chunjiang Yin, Hermann Rohling: Convergence Manager: flexible use of wireless standards. Proceedings 9th international OFDM workshop, Dresden, Germany, September 2004.

  - Lars Ohliger, Nico Tönder, Marc Reinert, Hermann Rohling: Configuration and Monitoring Concept for a FPGA based OFDM Demonstrator. Proceedings 9th international OFDM workshop, Dresden, Germany, September 2004.

  - Christian Stimming, Hermann Rohling: Evaluation of MIMO-OFDM and MIMO channel models in the Open-Source Simulation Software Simthetic. Proceedings 9th international OFDM workshop, Dresden, Germany, September 2004.

  - Hermann Rohling, Rainer Grünheid: Cross Layer Considerations for an Adaptive OFDM-Based Wireless Communication System, Wireless Personal Communicatons, Kluwer Academic Publishers.

  - Myles Capstick, Ben Kemp, Alister Burr, Christian Stimming: Multiband MIMO antenna arrays. 13th IST MWCS, Lyon, France, 28. –30. June 2004.

  - Nico Tönder, Marc Reinert, Josef Eichinger, Hermann Rohling: Optimal Channel Code Parameter Selection for Hardware Efficient OFDM Systems. Proceedings. IEEE VTC’04 Fall, Los Angeles, USA, 2004.

  - André Ebner, Lars Wischhof, Hermann Rohling: Aspects of Decentralized Time Synchronization in Vehicular Ad hoc Networks. Proceedings of the 1st International Workshop on Intelligent Transportation (WIT 2004), Hamburg, Germany, March 2004.

  - Hermann Rohling, Dirk Galda: An OFDM based Cellular Single Frequency Communication Network. Proceedings of WWRF, Beijing, 23./24.February 2004

  - Wilfried Enkelmann, Lars Wischhof, André Ebner, Hermann Rohling: FleetNet – Anwendungen für mobile Ad-hoc-Netzwerke. Praxis der Informationsverarbeitung und Kommunikation, Heft 4, 26. Jahrgang, Seiten 197-202, December 2003.

  - Mattias Lampe, Tobias Giebel, Hermann Rohling, Wolfgang Zirwas: PER Prediction for PHY Mode Selection in OFDM Systems. Proc. Globecom 2003, San Francisco, USA, December 2003

  - Wolfgang Zirwas, Mattias Lampe, Hui Li, Matthias Lott, Martin Weckerle, Egon Schulz: Radio Resource Management in Cellular Multihop Networks. Proc. MoMuC 2003, München, October 2003

  - Rainer Grünheid, Bing Chen, Hermann Rohling: Joint Layer Design for an Adaptive OFDM Transmission System. Proc. IEEE VTC’03 Fall, Orlando, Florida, USA, October 2003.

  - Sonom Olonbayar, Hermann Rohling, Tobias Giebel, Rainer Gruenheid: Decision Directed Channel Estimation in OFDM Based Systems. Proc. 8th International OFDM Workshop, S.162-166, Hamburg, September 2003.

  - Mark Schiementz, Florian Fölster, Hermann Rohling. Angle Estimation Techniques for different 24 GHz Radar Networks. Proc. IRS2003 Dresden, Germany, September 2003.

  - Mattias Lampe, Tobias Giebel, Hermann Rohling, Wolfgang Zirwas. Signalling-Free Detection of Adaptive Modulation in OFDM Systems. Proc. PIMRC 2003, Peking, China, September 2003.

  - Bing Chen, Hermann Rohling. Joint Optimization of DLC Layer and PHYsical Layer in an OFDM-based Communication System. Proc. 8th International OFDM Workshop, Hamburg, September 2003.

  - Hermann Rohling, Rainer Grünheid, Dirk Galda. OFDM Transmission Technique for the 4th Generation of Mobile Communication Systems, Proc. of the Int. Symposium on Telecommunications (IST) 2003, Isfahan, Iran, August 2003.

  - Dirk Galda, Hermann Rohling, Elena Costa. On the Effects of User Mobility on the Uplink of an OFDMA System. Proc. IEEE VTC'03 Spring, Jeju, South Korea, April 2003.

  - Jianjun Ran, Rainer Grünheid, Hermann Rohling, Edgar Bolinth, Ralf Kern. Decision-directed Channel Estimation Method for OFDM systems with high velocities. Proc. IEEE VTC'03 Spring, Jeju, Korea, April 2003.

  - André Ebner, Hermann Rohling, Lars Wischhof, Rüdiger Halfmann, Matthias Lott. Performance of UTRA TDD ad hoc and IEEE 802.11b in Vehicular Environments. Proc. IEEE VTC'03 Spring, Jeju.

  - F. Adelantado, O. Sallent, J. Pérez-Romero, R. Agustí, Impact of Traffic Hotspots in 3G W-CDMA Networks, IEEE Semiannual Vehicular Technology Conference (VTC 2004-Spring), Milan, May 2004.

  - S. Plass and S. Kaiser, MC-CDMA versus OFDMA in Cellular Environments (invited paper), 13th European Signal Processing Conference (EUSIPCO 2005), Antalya, Turkey, Sept. 2005.

  - S. Plass, A. Dammann, and S. Kaiser, Error Performance for MC-CDMA and OFDMA in a Downlink Multi-Cell Scenario,  IST Mobile and Wireless Communications Summit 2005, Dresden, Germany, June 2005.

  - S. Plass, A. Dammann, and S. Kaiser, Analysis of Coded OFDMA in a Downlink Multi-Cell Scenario, 9th Int. OFDM-Workshop (InOWo 2004), Dresden, Germany, September 2004.

  - S. Plass, S. Sand, and G. Auer, Modeling and Analysis of a Cellular MC-CDMA Downlink System, IEEE Int. Symp. on Personal, Indoor, and Mobile Radio Commun. (PIMRC 2004), Barcelona, Spain, September 2004.

  - Longauer, L.-Čížová, J.-Marchevský, S.- Kocur, D.: MSF-MUD and BA-MUD Receivers: Principles and Comparison. The Second IEEE International Conference on Computational Cybernetics. Proceedings. Vienna University of Technology, Vienna, Austria, August 30-September 1, 2004, pp. 453-457.

  - Grega, M.-Marchevský, S.-Kocur, D.: Packet Oriented Services Provided through Satellites. (in Slovak). Proceedings of COFAX-Telekomunikácie. 19.-20.4.2004, Bratislava 2004, p.113-116.

  - Čížová, J.-Kocur, D.-Marchevský, S.: Performance of the Microstatistic Multi-User Receiver for the Pass-Band DS-CDMA Transmission System. Proceeding of the 6th International Conference on New Trends in Aviation Development. 19.9.2004, Košice, Slovensko, ISBN 80-7166-050-7, p. 9-14.

  - Krajňák, J.-Kocur, D.: Multi-Carrier Modulations: Basics and Fundamentals. Proceeding of the 6th International Conference on New Trends in Aviation Development. 19.9.2004, Košice, Slovensko, ISBN 80-7166-050-7, p. 15-19.

  - Kocur, D.-Čížová, J.-Marchevský, S.: Adaptive Multi-Channel Microstatistic Filters. 48. Internationales Wissenschaftliches Kolloquium. Tagungsband. September 22-25, 2003, Germany. Technische Universität Ilmenau (Thűr.), 2003, pp.91-92. Rozšírené znenie príspevku je publikované v e-forme na CD nosiči: 48. Internationales Wissenschaftliches 22.-25.09.2003. Proceedings and our Portrait. Technische Universität Ilmenau, 2003.

  - Kocur, D.-Čížová, J.-Marchevský, S.: Microstatistic Multi-User Detection Receiver. IEEE International Conference on Computational Cybernetics. Proceedings. Siofok, Hungary, August 29-31, 2003, pp. 363-366.

  - Marchevský, S. - Kocur, D. - Longauer, L. - Čížová, J.: Simulation of Adaptive Blind Multi-User Detection of CDMA Signals by System Design Tool-System View. Recent Trends in Multimedia Information Processing. Proceedings of the 10th International Workshop on Systems, Signals and Image Processing, September 10-11, 2003, Sdělovací technika spol. s.r.o., Prag, Czech Republic, pp.203-206.

  - R. Tomé, J.L. Pijoan, C. Vilella, M. Deumal, “Método de la Comparación de Fases (MCF) para la reducción del Peak-to-Average Power Ratio (PAPR) en sistemas OFDM”, URSI, Sept. 2003, La Coruña.

  - M. Deumal, C. Vilella, P. Bergadà, J.L. Pijoan, J.R. Regué, “Plataforma Software Radio”, URSI, Sept. 2003, La Coruña.

  - C. Vilella, J.L. Pijoan, "An efficient high performance equalization method for real-valued modulations", Nordic Radio Symposium 2004, 16 –18 August 2004, Oulu, Finland.

  - M. Deumal, C. Vilella, J.L. Pijoan, P. Bergadà, "Partially Clipping Method for the Peak-to-Average Power Ratio (PAPR) reduction in OFDM", 15th IEEE International Symposium on Personal, Indoor and Mobile Radio Communications (PIMRC 2004), 5-8 September 2004, Barcelona, Spain.

  - I.Gutierrez, J.L. Pijoan, C. Vilella, "Caracterización del canal ionosférico en la Antártida", URSI 2004, Sept. 2004, Barcelona.

  - Gutierrez, D. Bartolomé, C. Vilella, "Estudio de los diferentes métodos de Control de Potencia para CDMA basados en Teoría de Juegos", URSI 2004, Sept. 2004, Barcelona.

  - C. Vilella, J.C. Socoró, D. Miralles, J.L. Pijoan, P. Bergadà, “HF channel measurements for digital communications from Antarctica”, 11th International Ionospheric Effects Symposium, May 2005, Alexandria, USA.

  - S. Bozay and M. Şafak, Performance analysis of spatial multiplexing and maximal-ratio combining in polarization diversity, 11th Signal Processing and Communications Applications Conference, Koç University, Istanbul, 18-20 June 2003, pp.273-276.

  - Rafet Yücel and Mehmet Şafak, Reduction of peak to average power ratio in OFDM systems by the method of improved maximum symbol control, IEEE 12th Signal Processing and Communications Applications Conference, Kuşadası, 28-30 April 2004, pp. 65-67.

  - Müge Yılmaz and Mehmet Şafak, Performance analysis of space-time coded CDMA Systems, Proc. 2nd National URSI Conference, September 2004, Bilkent University, Ankara

  - Rafet Yücel and Mehmet Şafak, Reduction of the peak to average power ratio in OFDM systems by the combined use of compandıng and clipping, Proc. 2nd National URSI Conference, September 2004, Bilkent University, Ankara.

  - S. Sand, R. Raulefs, and G. Auer, Iterative Channel Estimation for MC-CDMA. Vehicular Technology Conference Spring (VTC-Spring’05), Stockholm, Sweden, May/ June 2005.

  - S. Sand, R. Raulef, and G. Auer, Iterative Channel Estimation for High Mobility Broadband MC-CDMA Systems, International Conference on Communications (ICC’05), Seoul, Korea, May 2005.

  - S. Sand, A. Dammann, and G. Auer, Adaptive Pilot Symbol Aided Channel Estimation for OFDM Systems, 4. International Workshop on Multi-Carrier Spread-Spectrum (MC-SS 2003), Oberpfaffenhofen, Germany, September 2003, pp. 227-234.

- D. Kocur, J. Krajnak, S. Marchevsky, "Piece-Wise Linear Multi-Channel Complex Microstatistics Filters", The 10th IEEE International Conference on Intelligent Engineering Systems 2006 (INES‘2006), London, United Kingdom, June 26-28, 2006.

- Cízova, J.- Kocur, D. - Marchevsky, S., "Multi-channel Microstatistic Filter Design For Microstatistic Multi-user Receiver", The 6th International Scientific Conference on Digital Signal Processing and Multimedia Communications. Košice, Slovakia, September 27-29, 2005. ISBN 80-7166-050-7, s. 114-117.

- Kocur, D.- Cízova, J.- Marchevsky, S., "Sub-Optimum MSF-MUD for CDMA Systems", The 2nd COST 289 Workshop on Special Topics on 4G Technologies. Turkey, Antalya, July 6-7, 2005, pp. 65-71.

- Copjan, L. - Marchevsky, S. - Kocur, D. - Benco, S., "Turbo Blind Multiuser Detection for CDMA", 6th International Scientific Conference on Digital Signal Processing and Multimedia Communications. Košice, Slovakia, September 27-29, 2005. ISBN 80-7166-050-7, s. 106-109.

- Pavelka, P. - Galajda, P., "Implementing a Software Defined Radio (SDR) Blocks on Reconfigurable Architecture", 6th DSP-MCOM 2005. International Conference on Digital Signal Processing and Multimedia Communications, September 13-14, 2005, Košice, Slovakia, pp.170-173. (ISBN 80-8072-323-6).

- Galajda, P. - Marchevsky, S. - Kocur, D. - Benco, S., "Mobile Learning at the Technical University of Košice", Proceedings of the 9th IEEE International Conference on Intelligent Engineering Systems 2005 (INES’2005), Cruising on Mediterranean Sea, September 16-19, 2005, pp. 233-237.

- Palubova, H., "OFDM System for Broadband Communication", 6th PhD Student Conference and Scientific and Technical Competition of Students of Faculty of Electrical Engineering and Informatics Technical University of Košice, 17.5.2006, Košice, Slovakia, pp. 101-102. (ISBN 80-8086-035-1).

- Pavelka, P., "Reducing the Peak- to- Average Power Ration (PAPR) in OFDM", The 6th PhD Student Conference and Scientific and Technical Competition of Students of  Faculty of Electrical Engineering and Informatics Technical University of Košice, 17.5.2006, Košice, Slovakia, pp. 103-104. (ISBN 80-8086-035-1).

- Grega, M.-Copjan, L.-Marchevsky, S.-Benco, S., "Simulation of Satellite Networks in NS-2 Environment", The 5th EURASIP Conference focused on Speech and Image Processing, Multimedia Communications and Services (EC-SIP-M 2005), June 29-July 2, 2005, Smolenice, Slovakia, pp.407-412.

- Grega, M.- Copjan, L.- Marchevsky, S.- Benco, S., "Possibility of Using Network Simulator (NS-2) for Modelling Satellite Networks", Acta Electrotechnica et Informatica, No.4, Vol.5, 2005, pp.35-38.

- Copjan, L.-Marchevsky, S., "Effective transmission over AWGN channel using Turbo codes", Junior Scientist Conference 2006- Science and Technology for The Future. Vienna, Austria, Vienna University of Technology, April 19 -21, 2006.

- Dobos, L. - Duha, J. - Marchevsky, S. - Wieser, V., "Mobilni radiomrezi. (Mobile Radio Networks, monograph in Bulgarian language)", Sofia, Bulgaria, 2005, pp.382.

- V. Balakirsky and A.J. Han Vinck, "Potential Performance of PLC Systems Composed of Several Communication Links", 9th International Symposium on Power-Line Communications and Its Applications, Vancouver, April 6-8, 2005, pp. 12-16, ISBN 0-7803-8844-5

- Jaco Versfeld and A.J. Han Vinck, "Reed-Solomon Coding to Enhance the reliability of M-FSK in a Power Line Environment", 9th International Symposium on Power-Line Communications and Its Applications, Vancouver, April 6-8, 2005, pp.100-104, ISBN 0-7803-8844-5

- Jaco Versfeld and A.J. Han Vinck, "Simulation Results for Permutation Trellis Codes using M-ary FSK", 9th International Symposium on Power-Line Communications and Its Applications, Vancouver, April 6-8, 2005, pp. 317-321, ISBN 0-7803-8844-5

- K. Ouahada, H.C. Ferreira, A.J. Han Vinck, L. Cheng, "On Combined Spectral shaping and M-FSK Modulation for Power Line Communications", 9th International Symposium on Power-Line Communications and Its Applications, Vancouver, April 6-8, 2005, pp. 351-355, ISBN 0-7803-8844-5

- A.J. Han Vinck, "Coding for Non-AWGN channels", Proc. 8-th Int. Symposium on Communications Theory and Applications, Ambleside, UK, pp.258-261, 2005, ISBN 0-85316-2441

- V. Balakirsky and A.J. Han Vinck, "Coding Aspects of Data Transmission over Bus Systems", roc. 8-th Int. Symposium on Communications Theory and Applications, Ambleside, UK, pp. 380-385, 2005, ISBN 0-85316-2441

- A.J. Han Vinck, "Coding for a Terrible Channel", EU-COST289 2nd  Workshop, Special Topics on 4G Technologies, Antalya, July 6-7, 2005, Turkey, pp. 101-106

- Anil Mengi, Gerhard Bauch, A.J. Han Vinck, "Space-Time differentially Coded Orthogonal Matix Modulation using QAM", EU-COST289 2nd Workshop, Special Topics on 4G Technologies, Antalya, July 6-7, 2005, Turkey, pp. 119-125

- Yanling Chen and A.J. Han Vinck, "Notes on Refined Fibonacci Numbers", Proceedings Winterschool on coding and Information theory, Bratislava, Slovakia, Febr. 20-25, 2005, ISBN 3-902477-01-6, pp. 41-46

- T. Imoto, N. Suehiro, A.J. Han Vinck, M. Nakamura, C. Han, "Analysis of information transmission using additional chip-shift signal with parallel pilot signal for multipath estimation based on complete complementary sequences", IWSDA2005, Shimonoseki, Japan, October 10-14

- D. Bajic, C. Stefanovic, “Sequences and Survival Probability”, Proceedings of IEEE TELSIKS 2005 - the 7th International Conference on Telecommunications in Modern Satellite, Cable and Broadcasting Services, 28 – 30 September 2005, Niš, Serbia and Montenegro

- D. Vukobratovic, M. Despotovic, “On the Packet Lengths of Rateless Codes”, Proceedings of the IEEE EUROCON 2005, Belgrade, pp. 672-676, November 2005, Serbia & Montenegro.

- C. Stefanovic, D. Bajic, “Search Process in Non-equiprobable Data”, Proceedings of the IEEE EUROCON 2005, 21 – 24 November 2005, Belgrade, Serbia and Montenegro.

- T. Willink, D. Bajic, P. Kovjanic, C. Stefanovic, "On Criteria for Short Acquisition Sequences Choice", Proceedings of the IEEE EUROCON 2005, 21 – 24 November 2005, Belgrade, Serbia and Montenegro.

- C. Stefanovic, D. Bajic, “On Optimization of Frame Lengths and Frame Delimiting Patterns for Data Communications Using Bifix Approach”, Proceedings of the IEEE EUROCON 2005, 21 – 24 November 2005, Belgrade, Serbia and Montenegro.

- P. Vontobel, R. Smarandache, N. Kiyavash, J. Teutsch, D. Vukobratovic, “On the Minimal Pseudo-Codewords of Codes from Finite Geometries”, Proceedings of the International Symposium of Information Theory ISIT 2005, pp. 980-985, September 2005, Adelaide, Australia.

- D. Bajic, C. Stefanovic and D. Vukobratovic, "Search Process and Probabilistic Bifix Approach", Proceedings of International Symposium on Information Theory ISIT2005, Adelaide, Australia, September 2005.

- D. Vukobratovic, “On the equivalence of BP and LP Decoding on the binary erasure channel”, Proceedings of 4th International Workshop on Optimal Codes and Related Topics 2005, pp. 275-282, June 2005, Pamporovo, Bulgaria.

- D. Bajic, C. Stefanovic, "Low Power Consuming, FEC Scheme" Proceedings of International Workshop on Optimal Codes and Related Topics 2005, 17-23.6 2005, Pamporovo, Bulgaria, June 2005.

- J. Stojanovic, D. Bajic, I. Stanojevic, "Throughput analysis of statistically synchronizable cyclic codes", Proceedings of Symposium on trends in Communications SympoTYC’04, Bratislava, Slovakia, 24-26 October, 2004, pp. 122 – 125

- D. Bajic, J. Stojanovic, "Analytical approach to the duration of search for different patterns", Proceedings of Symposium on trends in Communications SympoTYC’04, Bratislava, Slovakia, 24-26 October, 2004, pp. 126 – 129

- D. Bajic, “On Survival Probability of Alignment Sequences”, International Symposium on Information Theory and its Applications, ISITA 2004, Parma, Italy, October 10–13, 2004, pp 1146-1151, ISBN: 4-902087-08-1

- V. Senk, D. Vukobratovic, I. Djurdjevic, “A new method of BER estimation for iteratively decodable codes”, Proceedings of International Symposium of Information Theory and Applications ISITA 2004, pp. 27-30, October 2004, Parma, Italy.

- D. Bajic, A. Burr, “A Simple Suboptimal Integer Code”, International Symposium on Information Theory and its Applications, ISITA 2004, Parma, Italy, October 10–13, 2004, pp 1315-1320., ISBN: 4-902087-08-1

- D. Bajic, J. Stojanovic, “Distributed Sequences and Search Process”. IEEE International Conference on Communications ICC2004, Paris, France, June 2004, CT08-6, ISBN: 0-7803-8534-9

- D. Bajic, J. Stojanovic, “An Analysis of the Search Process for Different Patterns in Random Data”, Proceedings of IEEE Region 8 EUROCON 2003 The International Conference, Ljubljana, Slovenia, September 2003, ISBN: 0-7803-7763-X, PR-2-2.

- D. Bajic, J. Stojanovic, J. Lindner, “Multiple window-sliding search”, Proceedings of ISIT 2003, Yokohama, Japan, June 2003. pp. 249

- D. Bajic, J. Stojanovic, “Search process for a fixed pattern with errors in random data: an analysis”, Proceedings of IEEE TELSIKS 2003 the international conference, Niš, Serbia and Montenegro, October 2003, pp 526-529.

- J. Joaquín Escudero Garzás, Ana García Armada, "QoS-Aware Scheduling for Wireless Sensor Networks", 15th IST Mobile & Wireless Communications Summit, Myconos, Greece, June 2006.

- J. Joaquín Escudero Garzás, Ana García Armada, “Analysis of the Performance of Energy-Efficient Scheduling for Wireless Sensor Networks Using SNR Gap”, IEEE Workshop on Smart Antennas, WSA 2006, Reisensburg, Germany, March 2006.

- M. Sánchez-Fernández, A. Lozano, “Doppler Sensitivity of Link Reciprocity in TDD MIMO Systems”, Proc. IEEE Globecom 2005, St. Louis, USA, December 2005.

- Víctor P. Gil Jiménez, Ana García Armada, “Multi-user synchronization in ad-hoc environments for OFDM-based Wireless Personal Area Networks”, Internacional Symposium on Wireless Personal Multimedia Communications. IWS / WPMC. Aalborg. Denmark. September, 2005

- Beatriz Bardón, Isaac Seoane, Víctor P. Gil Jiménez, Matilde Sánchez Fernández, Ana García Armada, “Discrete Channel Models for IEEE 802.11a”, Internacional Symposium on Wireless Personal Multimedia Communications. IWS / WPMC. Aalborg. Denmark. September, 2005.

- Khalid El Baamrani, Víctor P. Gil, Ana García Armada, A. A. Ouahman, Said el Allaki, “Low-Complexity Multiuser Bit-Loading Algorithm for the Downlink of Wireless Local Area Networks”, IEEE Vehicular Technology Conference VTC Spring. Stockholm (Sweden). June, 2005.

- A. Wittneben and I. Hammerström, "Multiuser Zero Forcing Relaying with Noisy Channel State Information", IEEE Wireless Communications and Networking Conference, WCNC 2005, Mar. 2005.

- S. Berger and A. Wittneben, "Experimental Performance Evaluation of Multiuser Zero Forcing Relaying in Indoor Scenarios", IEEE Vehicular Technology Conference, VTC Spring 2005, May 2005.

- A. Wittneben, "A Theoretical Analysis of Multiuser Zero Forcing Relaying with Noisy Channel State Information", IEEE Vehicular Technology Conference, VTC Spring 2005, May 2005.

- M. Kuhn and A. Wittneben, "Wireless Relaying with Partial Cooperation Based on Power-Line Communication", 6th IEEE International Workshop on Signal Processing Advances in Wireless Communications, SPAWC 2005, New York, June 2005.

- Hammerström, J. Zhao, and A. Wittneben, "Temporal Fairness Enhanced Scheduling for Cooperative Relaying Networks in Low Mobility Fading Environments", 6th IEEE International Workshop on Signal Processing Advances in Wireless Communications, SPAWC 2005, New York, June 2005.

- E. Auger, B. Rankov, M. Kuhn, A. Wittneben, "Time Domain Precoding for MIMO-OFDM Systems", OFDM Workshop, Hamburg, Germany, Aug. 2005.

- B. Rankov and A. Wittneben, "Spectral Efficient Protocols for Nonregenerative Half-duplex Relaying", Allerton Conference on Communication, Control, and Computing, Monticello, IL, Sept. 2005.

- I. Hammerström, J. Zhao, S. Berger, and A. Wittneben, "Experimental Performance Evaluation of Joint Cooperative Diversity and Scheduling", IEEE Vehicular Technology Conference, VTC Fall 2005, Dallas, Sept. 2005.

- B. Rankov and A. Wittneben, "Spectral Efficient Signaling for Half-duplex Relay Channels", Asilomar Conference on Signals, Systems, and Computers 2005, Pacific Grove, CA, Nov. 2005.

- S. Berger and A. Wittneben, "Cooperative Distributed Multiuser MMSE Relaying in Wireless Ad-Hoc Networks", Asilomar Conference on Signals, Systems, and Computers 2005, Pacific Grove, CA, Nov. 2005.

- A. Wittneben, "Coherent Multiuser Relaying with Partial Relay Cooperation", IEEE Wireless Communication and Networking Conference, Las Vegas, NV, USA, Apr. 2006.

- I. Hammerström and A. Wittneben, "Joint Power Allocation for Nonregenerative MIMO-OFDM Relay Links", International Conference on Acoustics, Speech, and Signal Processing (ICASSP), Toulouse, France, May 2006.

- I. Hammerström and A. Wittneben, "On the Optimal Power Allocation for Nonregenerative OFDM Relay Links", Proc. IEEE International Conf. on Communications (ICC), Istanbul, Turkey, 11-15 June 2006.

- C. Mensing and S. Plass, "Positioning algorithms for cellular networks using TDOA", Proceedings IEEE International Conference on Acoustics, Speech, and Signal Processing (ICASSP 2006), Toulouse, France, May 2006.

- I. Groh, S. Plass, and S. Sand, "Capacity approximation for uncorrelated MIMO channels using random matrix methods", Proceedings Second International Symposium on Communications, Control and Signal Processing, (ISCCSP 2006), Marrakech, Morocco, 13-16 March 2006.

- S. Plass and A. Dammann, "A partitioned cellular environment for MC-CDMA and OFDMA", Proceedings 15th Wireless World Research Forum (WWRF), Paris, France, Dec. 2005.

- S. Plass and A. Dammann, "On the error performance of sectorized cellular systems for MC-CDMA and OFDMA", Proceedings IEEE International Symposium on Personal, Indoor and Mobile Radio Communications (PIMRC 2005), Berlin, Germany, Sept. 2005.

- S. Plass and A. Dammann, "Radio resource management for MC-CDMA over correlated Rayleigh fading channels", Proceedings 5th International Workshop on Multi-Carrier Spread-Spectrum (MC-SS 2005), Oberpfaffenhofen, Germany, Sept. 2005.

- C. Vilella, J.L. Pijoan, "An efficient high performance equalization method for real-valued modulations", Nordic Radio Symposium 2004, 16 -18 August 2004, Oulu, Finland

- M.Deumal, C.Vilella, J.L.Pijoan, P.Bergadà, "Partially Clipping Method for the Peak-to-Average Power Ratio (PAPR) reduction in OFDM", 15th IEEE International Symposium on Personal, Indoor and Mobile Radio Communications (PIMRC 2004), vol.1, pp.464-468, 5-8 September 2004, Barcelona, Spain.

- C.Vilella, J.C.Socoró, D.Miralles, J.L.Pijoan, P.Bergadà , “HF channel measurements for digital communications from Antarctica”, 11th International Ionospheric Effects Symposium IES 2005, Alexandria, Virginia, USA, 3-5 May 2005.

- C.Vilella, D.Miralles, J.C.Socoró, J.L.Pijoan, R.Aquilué, "A new sounding system for HF digital communications from Antarctica", 2005 International Symposium on Antennas and Propagation (ISAP2005), 3-5 August 2005, Seoul, Korea.

- I.Gutiérrez, F. Bader, J.L.Pijoan, M. Deumal, "Performance of a new adaptive grouping and modulation procedure for multiuser group orthogonal MC-CDMA", IASTED Communication Systems and Networks (CSN), 12-14 September 2005, Benidorm, Spain.

- I.Gutierrez, J.L.Pijoan, M.Deumal, F.Bader, “Adaptive Bit Loading with Multi-User Diversity in MC-CDMA”, Proceedings of European Wireless 2006, April 2006, Athens.

- I.Gutierrez, J.L.Pijoan, R.Aquilué, F.Bader, “New Channel Interpolation Method for OFDM Systems by Nearest Pilot Padding”, Proceedings of European Wireless 2006, April 2006, Athens.

- M.Deumal, J.L. Pijoan, I.Gutiérrez, A.Behravan, "Peak reduction of multi-carrier systems by Controlled Spectral Outgrowth", Proc. IEEE International conference on Acoustics, Speech and Signal Processing (ICASSP 2006), May 2006, Toulouse.

- M.Deumal, C.Vilella, J.C.Socoró, R.M.Alsina, J.L.Pijoan, "A DS-SS signaling based system proposal for low SNR HF digital communications", Proc. Ionospheric Radio Systems and Techniques, July 2006, London.

- R.Aquilué, P.Bergadà, I.Gutiérrez, J.L.Pijoan, "Channel Estimation for Long Distance HF communications based on OFDM Pilot Symbols", Proc. Ionospheric Radio Systems and Techniques, July 2006, London.

- C.Vilella, J.C.Socoró, J.L.Pijoan, I.Gutiérrez, D.Altadill, "An Antarctic to Spain HF Link. Oblique Sounding Results", Proc. Ionospheric Radio Systems and Techniques, July 2006, London.

- C.Vilella, P.Bergadà, M.Deumal, J.L.Pijoan, R.Aquilué, "Transceiver Architecture and Digital Down Converter Design for a Long Distance, Low Power HF Ionospheric Link", Proc. Ionospheric Radio Systems and Techniques, July 2006, London.

- C.Vilella, D.Badia, J.L.Pijoan, M.Deumal, M.Ribo, J.R.Regué, "On site receiver testing. Application to long distance HF links", EMC Europe 2006, September 2006, Barcelona.

- Tahmasebi Toyserkani, A., Ström, E. G., Svensson, A., "An Efficient Broadcast MAC Scheme for Traffic Safety Applications in Automotive Networks", Proceedings IEEE Wireless Communications & Networking Conference, 2006, Las Vegas, Nevada, USA. CPL 20174

- Munier, F., Wessman, M-O., Eriksson, T. et al., "On the Effect of Antennas on UWB Systems", Proceedings URSI General Assembly, New Delhi, India. CPL 11995, 2005

- Rydström, M., Urruela, A., Ström, E. G. et al., "A Low Complexity Algorithm for Sensor Localization", Proceedings of the 11th European Wireless conference, Nicosia, Cyprus. CPL 11982, 2005

- Rydström, M., Ström, E. G., Svensson, A., "Clock-offset Cancellation Methods for Positioning in Asynchronous Sensor Networks", Proceedings of the IEEE WirelessCom, Hawaii, USA. CPL 11991, 2005

- Rydström, M., Urruela, A., Ström, E. G. et al., "Practical Automotive Applications of Cramer-Rao Bound Analysis", Proceedings of the IEEE Intelligent Vehicles symposium, Las Vegas, Nevada, USA. CPL 11986, 2005

- Rydström, M., Tahmasebi Toyserkani, A., Ström, E. G. et al., "Towards a Wireless Network for Traffic Safety Applications", Proceedings RadioVetenskap och Kommunikation, Linköping, Sweden. pp. 375-380. CPL 11981, 2005

- Wang, W., Ottosson, T., Svensson, T. et al., "Evaluations of a 4G Uplink System Based on Adaptive Single-Carrier TDMA", Proceedings IEEE Vehicular Technology Conference Fall, Dallas, Texas, USA. CPL 11996, 2005

- Wessman, M-O., Svensson, A., Agrell, E., "Performance of Coded Single-Band Carrier-Based DS-SS Systems on IEEE 802.15.3a UWB Channels", Proceedings IEEE International Conference on Ultra-Wideband, Zürich, Switzerland. pp. 152-157. CPL 11992, 2005

- I. Safak and M. Uner, Turbo decoding on the TMS320C6416 digital signal processor,  Proc. 2006 IEEE 14th Signal Processing and Communication Applications Conference, 17-19 April 2006, Antalya, Turkey.